- فصل اول) ویژگیهای طرح

1-1-شناسایی

- اسکلت سازه از نوع بتن آرمه میباشد.

- سیستم مقاوم جانبی سازه در هر دو جهت قاب خمشی با شکلپذیری متوسط میباشد.

- دیافراگم صلب طبقات، دال یکطرفه از نوع تیرچه و بلوک میباشد.

- فونداسیون سازه از نوع نواری میباشد.

- پروژه شامل 3 طبقه سازهای میباشد که کاربری طبقه همکف پارکینگ و انباری بوده و بقیه طبقات کاربری مسکونی داشته

- در طبقات کف سازی به همراه ارتفاع سقف مجموعا 40 سانتیمتر در نظر گرفته شده است.

1-2-مشخصات مصالح مصرفی

- مقاومت مشخصه بتن مصرفی سازه                                                        fc'=280 kg/cm2

- مقاومت نظیر حد تسلیم میلگردهای مصرفی در اسکلت و فونداسیون                    Fy=4000 kg/cm2AIII

- مقاومت نظیر حد تسلیم میلگردهای مصرفی در تیرچهها و خاموتها                        Fy=3000 kg/cm2AII

- مدول الاستیسیته بتن مصرفی                                                 Ec=2.76×105 kg/cm2

- مدول الاستیسیته فولاد مصرفی                                               Es=2.0×106 kg/cm2

3-1-معیارهای طراحی ( مراجع و آئین نامهها)

1-3-1-آئین نامههای بارگذاری

- مبحث ششم مقررات ملی ساختمان – بارهای وارد بر ساختمان ویرایش 1392

- آئین نامه طراحی ساختمانها در برابر زلزله استاندارد 2800 ویرایش چهارم

2-3-1-آئین نامههای طراحی

- آئین نامه طراحی سازههای بتن آرمه آمریکا ، ACI318-14

- مبحث نهم مقررات ملی ساختمان – طرح و اجرای سازههای بتن آرمه ویرایش 1392

- مبحث هفتم مقررات ملی ساختمان – پی و پی سازی ویرایش 1392

4-1-فهرست منابع و مراجع مورد استفاده

- کتاب بارگذاری سازهها – موسسه سری عمران

- طراحی سازههای بتن آرمه با ETABS – موسسه سری عمران

5-1-نرم افزارهای مورد استفاده

- طراحی سازه –2016  ETABS

- کنترلهای طراحی سازه – Excel 2016

2- فصل دوم( بارگذاری )

1-2- بارمرده

مطابق صورت پروژه بار مرده سقف، دیوار ، تیر و ستونها بصورت میانگین برابر 600 کیلوگرم بر متر مربع در نظر گرفته شده است.

2-2- بارهای  زنده

- بار زنده 350 کیلوگرم بر متر مربع میباشد.

- بار پارتیشن نیز 100 کیلوگرم بر متر مربع میباشد.

3-2- محاسبه بار برف

2 - 3 - 1 - بار برف زمین (Pg )

بار برف زمین با توجه به تقسیم بندی مشخص شده در شکل و جدول زیر بدست می آید :

شهر تهران در منطقه 4 قرار دارد بنابراین :

Pg = 150 kg / m²

2-7-2- ضریب اهمیت (Is)

برای ساختمانهای مسکونی ضریب اهمیت برف برابر 1 میباشد.

3-7-2- ضریب شیب (Cs)

بدلیل مسطح بودن سطح بام ضریب شیب برابر 1 می باشد.

4-7-2- ضریب یرف گیری (Ce)

ضریب برف گیری با در نظر گیری گروه ناهمواری محیط و نوع برف گیر بودن بام به شرح زیر تعیین می گردد :

گروه ناهمواری محیط :

گروه ناهمواری زیاد - محیط شهری و حومه شهری،محیط باغ ،جنگل و سایر محیط های شامل ناهمواری و موانع متعدد و متراکم با ارتفاع 9 متر یا بیشتر.

گروه ناهمواری کم - محیط مستوی بدون موانع از قبیل دریا و دریاچه، باتلاق و نمکزار، در نظر گرفتن چهار جهت باد متفاوت منطبق بر دو امتداد متعامد کافی می باشد.

ساختمان در داخل شهر قرار دارد و با توجه به موقعیت سازه نیمه برفگیر محسوب می شود بنابراین ضریب برفگیری برابر 1 می باشد.

2-7-5- بار برف بام (Pr)

بار برف بام با توجه به شیب و دمای بام، برفگیری و اهمیت سازه، برای هر متر مربع تصویر افقی سطح آن به صورت زیر تعیین می گردد:

𝑃𝑟 = 0.7 𝐶𝑠 𝐶𝑡 𝐶𝑒 𝐼𝑠 𝑃𝑔 = 105 kg / m²

4-2- بارگذاری زلزله

1-4-2- محاسبه نیروی برش پایه به روش استاتیکی معادل

1-1-4-2- ملاحظات کلی

- شهر محل احداث                                            تهران

- کاربری ساختمان                                             مسکونی

- تعداد سقفها از تراز پایه                                    8

- ارتفاع ساختمان از تراز پایه (متر)                          8/28

- منظم بودن ساختمان                                       سازه منظم است بنابراین از تحلیل استاتیکی استفاده می کنیم.

- لزوم اعمال از قاعده 100 -30                             بلی

با توجه به اینکه برای این سازه از سیستم باربر جانبی قاب خمشی متوسط بعلاوه دیوار برشی استفاده شده است و اتصالات تیر به ستون نیروی زلزله را تحمل خواهند کرد و در همه ستونها حداقل دو تیر که سیستم مقاوم جانبی بشمار میآیند متصل شده است که بنا به استاندارد 2800 بایستی این قاعده را نیز در اعمال ترکیبات بار لحاظ نمائیم.

- لزوم اعمال بار قائم زلزله                                    بلی

این بند یکی از قواعد جدید استاندارد 2800 ویرایش چهارم میباشد که برای اعمال این قاعده برای سازههایی که در حوزه نزدیک گسل قرار دارند علاوه بر زلزله افقی، زلزله قائم نیز بایستی اعمال گردد که آئین نامه 4 شرط برای اعمال این قاعده مشخص نموده است که اولین شرط برای سازههایی که در مناطق با لرزهخیزی خیلی زیاد قرار دارند بایستی این بند اعمال گردد که این سازه چون در تهران قرار دارد و تهران جزو مناطق با لرزهخیزی خیلی زیاد بشمار میآید بایستی بار قائم زلزله نیز اعمال گردد.

2-1-4-2- محاسبه ضریب اهمیت ساختمان (I)

ساختمانهای مسکونی جزو گروه 3 قرار دارد بنابراین ضریب اهمیت آنها برابر یک میباشد.

3-1-4-2- محاسبه نسبت شتاب مبنای طرح (A)

با در نظر گرفتن محل احداث ساختمان، نسبت شتاب مبنای طرح به شتاب ثقل، بر اساس میزان خطر لرزه خیزی به شرح جدول زیر تعیین می گردد :

تهران جزو مناطق با پهنه خطر نسبی خیلی زیاد بشمار میآیند بنابراین نسبت شتاب مبنای طرح این مناطق برابر 35/0 خواهد بود.

4-1-4-2- محاسبه زمان تناوب اصلی نوسان (T)

5-1-3-2- محاسبه زمان تناوب تجربی جهت X

Tx=0.05H0.75=0.05×28.80.75=0.621 sec

با توجه به این بند آئین نامه به ما موظف کرده از بین 25/1 برابر زمان تناوب تجربی و زمان تناوب تحلیلی که از نرم افزار بدست خواهیم آورد را برای طراحی سازه لحاظ کنیم و بنا به توصیه اکثر مراجع در این پروژه زمان تناوب تجربی در همین ابتدا به 25/1 ضرب شده است و در بخش های بعدی آنرا با زمان تناوب تحلیلی مقایسه                  و کنترل خواهیم نمود.

Tax=1.25×0.621=0.777 sec

6-1-4-2- محاسبه زمان تناوب تجربی جهت Y

با توجه به اینکه در این جهت نیز از سیست قاب خمشی بتنی متوسط استفاده شده است زمان تناوب این جهت نیز همانند جهت X خواهد بود.

Ty=0.05H0.75=0.05×28.80.75=0.621 sec

Ta,y=1.25×0.621=0.777 sec

7-1-4-2- محاسبه ضریب بازتاب ساختمان (B)

محاسبات مربوط به ضریب بازتاب ساختمان با در نظر گرفتن پارامترهای ظریب شکل طیف و نوع زمین بصورت زیر می باشد :

8-1-4-2- طبقه بندی نوع زمین

در این پروژه نوع زمین، نوع II مشخص گردیده است.

9-1-3-2- پارامترهای ضریب طیف شکل

مقدار ضریب (S)            5/1

مقدار ضریب (S0)          0/1

مقدار ضریب (T0)          1/0

مقدار ضریب (Ts)          5/0

10-1-4-2- محاسبه ضریب اصلاح طیف طرح (N)

11-1-4-2- محاسبه ضریب شکل طیف (B1)

حال با توجه به نوع خاک و زمان تناوب بدست آمده خواهیم داشت:

B1=(1.5+1)0.50.777=1.608

N=0.74-0.50.777-0.5+1=1.055

B=B1N=1.608*1.055=1.7

با توجه به اینکه در دو جهت تمامی پارامترها یکسان است ضریب B نیز یکسان میباشد.

12-1-4-2- محاسبه ضریب رفتار ساختمان (Ru)

با در نظر گیری نوع سیستم مقاوم در برابر نیروهای جانبی مقدار ضریب رفتار در جهت های اصلی ساختمان بصورت زیر بدست می آید:

با توجه به اینکه در دو جهت سازه از قاب خمشی بعلاوه دیوار برشی متوسط استفاده شده است ضریب رفتار برابر 6 میباشد.

Rux=6                                      Ruy=6

13-1-4-2- محاسبه ضریب زلزله (C)

Cx=Cy=ABIRu=0.35×1.7×16=0.0991

14-1-4-2- محاسبه ضریب (K)

kx=ky=0.5T+0.75=0.5×0.777+0.75=1.138

بعد از محاسبه ضریب زلزله به سراغ محاسبه وزن موثر لرزه ای ساختمان میرویم:

ای لرزه موثر وزن=مرده بار+زنده و برف بار از درصدی +بندی تیغه زنده بار

الف) بارهای مرده

الف-1) بارهای مرده ناشی از کف طبقات

طبقات کف مرده بار=مربع متر یک وزن×طبقه مساحت

W=8×600×1050=5040000 Kgf=5040 Tonf

الف-2) بارهای مرده ناشی از دیوار ها

نمادار دیوار مرده بار=مربع متر یک وزن×طول×ارتفاع

W=8×600×130×3.2=1996800 Kgf=1996.8 Tonf

الف-3) مجموع بارهای مرده

Dead=7036.8 Tonf

ب) بارهای زنده

ب-1) بار زنده ناشی از کف طبقات

طبقات کف بار=زنده بار حداقل×طبقه مساحت

W=8×350×1050=2940000 Kgf=2940 Tonf

پ) بار زنده تیغه بندی

بندی تیغه زنده بار=بندی تیغه معادل بار×طبقه مساحت

W=7×100×1050=735000 Kgf=735 Tonf

ت) بار برف

برف بار=برف بار×طبقه مساحت

W=1×105×1050=110250 Kgf=110.25 Tonf

بعد از محاسبه بارها حال وزن موثر لرزهای را بصورت زیر محاسبه مینمائیم:

w=Dead+Lpart+0.2Live+0.2Snow

w=7036.8+735+0.2×2940+0.2×110.25=8381.35 Tonf

حال بعد از محاسبه وزن موثر لرزهای و ضریب زلزله برش پایه بصورت زیر برابر است با:

Vu=0.0991×8381.35=830.64 Tonf

توزیع برش پایه در طبقات

وزن موثر لرزهای طبقات

| طبقات | وزن (Tonf) |
| --- | --- |
| اول | 1/1058 |
| دوم | 1/1058 |
| سوم | 1/1058 |
| چهارم | 1/1058 |
| پنجم | 1/1058 |
| ششم | 1/1058 |
| هفتم | 1/1058 |
| هشتم | 65/974 |

جدول توزیع برش طبقات

| vui | Fui | Wihik | hi(m) | wi (Tonf) | طبقات |
| --- | --- | --- | --- | --- | --- |
| 53/183 | 53/183 | 07/44631 | 8/28 | 65/974 | 8 |
| 68/354 | 15/171 | 76/41621 | 2/25 | 1/1058 | 7 |
| 29/498 | 61/143 | 89/34924 | 6/21 | 1/1058 | 6 |
| 99/614 | 7/116 | 94/28380 | 0/18 | 1/1058 | 5 |
| 52/705 | 53/90 | 24/22016 | 4/14 | 1/1058 | 4 |
| 77/770 | 25/65 | 49/15869 | 8/10 | 1/1058 | 3 |
| 9/811 | 13/41 | 94/10003 | 2/7 | 1/1058 | 2 |
| 59/830 | 69/18 | 68/4545 | 6/3 | 1/1058 | 1 |
|  |  | =201994 |  |  |  |

برش طبقه اول تقریبا برابر برش پایه بوده و این نشان دهنده درستی توزیع نیروی برشی در طبقات میباشد و اختلاف 05/0 موجود بخاطر گرد کردن اعداد در محاسبات میباشد.

8-2- ترکیبات بارگذاری

ترکیبات بارگذاری از خود نرم افزار ETABS و تنظیم آن بر روی آئین نامه ACI318-14 فراخوانی شده است.

| نام حالت بار | علامت |
| --- | --- |
| بار مرده | Dead |
| بار زنده کاهش نیافته | Live |
| بار معادل تیغه بندی | Live partition |
| بار زنده بام | L Roof |
| بار برف | Snow |
| بار اصلاح جرم موثر لرزهای | Wall |
| بار زلزله جهت X بدون خروج از مرکزیت | EX |
| بار زلزله جهت Y بدون خروج از مرکزیت | EX |
| بار زلزله جهت X بدون خروج از مرکزیت و با خروج از مرکزیت مثبت و منفی | EX All |
| بار زلزله جهت Y بدون خروج از مرکزیت و با خروج از مرکزیت مثبت و منفی | EY All |
| بار زلزله برای کنترل دریفت جهت X | EX Drift |
| بار زلزله برای کنترل دریفت جهت Y | EY Drift |

| TABLE: Load Combinations |  |  |  |
| --- | --- | --- | --- |
| Name | Load Case/Combo | Scale Factor | Type |
| UDCon1 | Dead | 1.4 | Linear Add |
| UDCon2 | Dead | 1.2 | Linear Add |
| UDCon2 | Live | 1.6 |  |
| UDCon2 | Snow | 0.5 |  |
| UDCon2 | LPart | 1.6 |  |
| UDCon3 | Dead | 1.2 | Linear Add |
| UDCon3 | Live | 1.6 |  |
| UDCon3 | Lroof | 0.5 |  |
| UDCon3 | LPart | 1.6 |  |
| UDCon4 | Dead | 1.2 | Linear Add |
| UDCon4 | Live | 1 |  |
| UDCon4 | Snow | 1.6 |  |
| UDCon4 | LPart | 1 |  |
| UDCon5 | Dead | 1.2 | Linear Add |
| UDCon5 | Live | 1 |  |
| UDCon5 | Lroof | 1.6 |  |
| UDCon5 | LPart | 1 |  |
| UDCon6 | Dead | 1.41 | Linear Add |
| UDCon6 | Live | 1 |  |
| UDCon6 | Snow | 0.2 |  |
| UDCon6 | LPart | 1 |  |
| UDCon6 | EX | 1 |  |
| UDCon7 | Dead | 1.41 | Linear Add |
| UDCon7 | Live | 1 |  |
| UDCon7 | Snow | 0.2 |  |
| UDCon7 | LPart | 1 |  |
| UDCon7 | EX | -1 |  |
| UDCon8 | Dead | 1.41 | Linear Add |
| UDCon8 | Live | 1 |  |
| UDCon8 | Snow | 0.2 |  |
| UDCon8 | LPart | 1 |  |
| UDCon8 | EXAll | 1 |  |
| UDCon9 | Dead | 1.41 | Linear Add |
| UDCon9 | Live | 1 |  |
| UDCon9 | Snow | 0.2 |  |
| UDCon9 | LPart | 1 |  |
| UDCon9 | EXAll | -1 |  |
| UDCon10 | Dead | 1.41 | Linear Add |
| UDCon10 | Live | 1 |  |
| UDCon10 | Snow | 0.2 |  |
| UDCon10 | LPart | 1 |  |
| UDCon10 | EY | 1 |  |
| UDCon11 | Dead | 1.41 | Linear Add |
| UDCon11 | Live | 1 |  |
| UDCon11 | Snow | 0.2 |  |
| UDCon11 | LPart | 1 |  |
| UDCon11 | EY | -1 |  |
| UDCon12 | Dead | 1.41 | Linear Add |
| UDCon12 | Live | 1 |  |
| UDCon12 | Snow | 0.2 |  |
| UDCon12 | LPart | 1 |  |
| UDCon12 | EYAll | 1 |  |
| UDCon13 | Dead | 1.41 | Linear Add |
| UDCon13 | Live | 1 |  |
| UDCon13 | Snow | 0.2 |  |
| UDCon13 | LPart | 1 |  |
| UDCon13 | EYAll | -1 |  |
| UDCon14 | Dead | 1.41 | Linear Add |
| UDCon14 | Live | 1 |  |
| UDCon14 | Snow | 0.2 |  |
| UDCon14 | LPart | 1 |  |
| UDCon14 | EXAll+0.3EY | 1 |  |
| UDCon15 | Dead | 1.41 | Linear Add |
| UDCon15 | Live | 1 |  |
| UDCon15 | Snow | 0.2 |  |
| UDCon15 | LPart | 1 |  |
| UDCon15 | EXAll+0.3EY | -1 |  |
| UDCon16 | Dead | 1.41 | Linear Add |
| UDCon16 | Live | 1 |  |
| UDCon16 | Snow | 0.2 |  |
| UDCon16 | LPart | 1 |  |
| UDCon16 | EXAll-0.3EY | 1 |  |
| UDCon17 | Dead | 1.41 | Linear Add |
| UDCon17 | Live | 1 |  |
| UDCon17 | Snow | 0.2 |  |
| UDCon17 | LPart | 1 |  |
| UDCon17 | EXAll-0.3EY | -1 |  |
| UDCon18 | Dead | 1.41 | Linear Add |
| UDCon18 | Live | 1 |  |
| UDCon18 | Snow | 0.2 |  |
| UDCon18 | LPart | 1 |  |
| UDCon18 | EYAll+0.3EX | 1 |  |
| UDCon19 | Dead | 1.41 | Linear Add |
| UDCon19 | Live | 1 |  |
| UDCon19 | Snow | 0.2 |  |
| UDCon19 | LPart | 1 |  |
| UDCon19 | EYAll+0.3EX | -1 |  |
| UDCon20 | Dead | 1.41 | Linear Add |
| UDCon20 | Live | 1 |  |
| UDCon20 | Snow | 0.2 |  |
| UDCon20 | LPart | 1 |  |
| UDCon20 | EYAll-0.3EX | 1 |  |
| UDCon21 | Dead | 1.41 | Linear Add |
| UDCon21 | Live | 1 |  |
| UDCon21 | Snow | 0.2 |  |
| UDCon21 | LPart | 1 |  |
| UDCon21 | EYAll-0.3EX | -1 |  |
| UDCon22 | Dead | 0.69 | Linear Add |
| UDCon22 | EX | 1 |  |
| UDCon23 | Dead | 0.69 | Linear Add |
| UDCon23 | EX | -1 |  |
| UDCon24 | Dead | 0.69 | Linear Add |
| UDCon24 | EXAll | 1 |  |
| UDCon25 | Dead | 0.69 | Linear Add |
| UDCon25 | EXAll | -1 |  |
| UDCon26 | Dead | 0.69 | Linear Add |
| UDCon26 | EY | 1 |  |
| UDCon27 | Dead | 0.69 | Linear Add |
| UDCon27 | EY | -1 |  |
| UDCon28 | Dead | 0.69 | Linear Add |
| UDCon28 | EYAll | 1 |  |
| UDCon29 | Dead | 0.69 | Linear Add |
| UDCon29 | EYAll | -1 |  |
| UDCon30 | Dead | 0.69 | Linear Add |
| UDCon30 | EXAll+0.3EY | 1 |  |
| UDCon31 | Dead | 0.69 | Linear Add |
| UDCon31 | EXAll+0.3EY | -1 |  |
| UDCon32 | Dead | 0.69 | Linear Add |
| UDCon32 | EXAll-0.3EY | 1 |  |
| UDCon33 | Dead | 0.69 | Linear Add |
| UDCon33 | EXAll-0.3EY | -1 |  |
| UDCon34 | Dead | 0.69 | Linear Add |
| UDCon34 | EYAll+0.3EX | 1 |  |
| UDCon35 | Dead | 0.69 | Linear Add |
| UDCon35 | EYAll+0.3EX | -1 |  |
| UDCon36 | Dead | 0.69 | Linear Add |
| UDCon36 | EYAll-0.3EX | 1 |  |
| UDCon37 | Dead | 0.69 | Linear Add |
| UDCon37 | EYAll-0.3EX | -1 |  |
| UDWal1 | Dead | 1.4 | Linear Add |
| UDWal2 | Dead | 1.2 | Linear Add |
| UDWal2 | Live | 1.6 |  |
| UDWal2 | Snow | 0.5 |  |
| UDWal2 | LPart | 1.6 |  |
| UDWal3 | Dead | 1.2 | Linear Add |
| UDWal3 | Live | 1.6 |  |
| UDWal3 | Lroof | 0.5 |  |
| UDWal3 | LPart | 1.6 |  |
| UDWal4 | Dead | 1.2 | Linear Add |
| UDWal4 | Live | 1 |  |
| UDWal4 | Snow | 1.6 |  |
| UDWal4 | LPart | 1 |  |
| UDWal5 | Dead | 1.2 | Linear Add |
| UDWal5 | Live | 1 |  |
| UDWal5 | Lroof | 1.6 |  |
| UDWal5 | LPart | 1 |  |
| UDWal6 | Dead | 1.41 | Linear Add |
| UDWal6 | Live | 1 |  |
| UDWal6 | Snow | 0.2 |  |
| UDWal6 | LPart | 1 |  |
| UDWal6 | EX | 1 |  |
| UDWal7 | Dead | 1.41 | Linear Add |
| UDWal7 | Live | 1 |  |
| UDWal7 | Snow | 0.2 |  |
| UDWal7 | LPart | 1 |  |
| UDWal7 | EX | -1 |  |
| UDWal8 | Dead | 1.41 | Linear Add |
| UDWal8 | Live | 1 |  |
| UDWal8 | Snow | 0.2 |  |
| UDWal8 | LPart | 1 |  |
| UDWal8 | EXAll | 1 |  |
| UDWal9 | Dead | 1.41 | Linear Add |
| UDWal9 | Live | 1 |  |
| UDWal9 | Snow | 0.2 |  |
| UDWal9 | LPart | 1 |  |
| UDWal9 | EXAll | -1 |  |
| UDWal10 | Dead | 1.41 | Linear Add |
| UDWal10 | Live | 1 |  |
| UDWal10 | Snow | 0.2 |  |
| UDWal10 | LPart | 1 |  |
| UDWal10 | EY | 1 |  |
| UDWal11 | Dead | 1.41 | Linear Add |
| UDWal11 | Live | 1 |  |
| UDWal11 | Snow | 0.2 |  |
| UDWal11 | LPart | 1 |  |
| UDWal11 | EY | -1 |  |
| UDWal12 | Dead | 1.41 | Linear Add |
| UDWal12 | Live | 1 |  |
| UDWal12 | Snow | 0.2 |  |
| UDWal12 | LPart | 1 |  |
| UDWal12 | EYAll | 1 |  |
| UDWal13 | Dead | 1.41 | Linear Add |
| UDWal13 | Live | 1 |  |
| UDWal13 | Snow | 0.2 |  |
| UDWal13 | LPart | 1 |  |
| UDWal13 | EYAll | -1 |  |
| UDWal14 | Dead | 1.41 | Linear Add |
| UDWal14 | Live | 1 |  |
| UDWal14 | Snow | 0.2 |  |
| UDWal14 | LPart | 1 |  |
| UDWal14 | EXAll+0.3EY | 1 |  |
| UDWal15 | Dead | 1.41 | Linear Add |
| UDWal15 | Live | 1 |  |
| UDWal15 | Snow | 0.2 |  |
| UDWal15 | LPart | 1 |  |
| UDWal15 | EXAll+0.3EY | -1 |  |
| UDWal16 | Dead | 1.41 | Linear Add |
| UDWal16 | Live | 1 |  |
| UDWal16 | Snow | 0.2 |  |
| UDWal16 | LPart | 1 |  |
| UDWal16 | EXAll-0.3EY | 1 |  |
| UDWal17 | Dead | 1.41 | Linear Add |
| UDWal17 | Live | 1 |  |
| UDWal17 | Snow | 0.2 |  |
| UDWal17 | LPart | 1 |  |
| UDWal17 | EXAll-0.3EY | -1 |  |
| UDWal18 | Dead | 1.41 | Linear Add |
| UDWal18 | Live | 1 |  |
| UDWal18 | Snow | 0.2 |  |
| UDWal18 | LPart | 1 |  |
| UDWal18 | EYAll+0.3EX | 1 |  |
| UDWal19 | Dead | 1.41 | Linear Add |
| UDWal19 | Live | 1 |  |
| UDWal19 | Snow | 0.2 |  |
| UDWal19 | LPart | 1 |  |
| UDWal19 | EYAll+0.3EX | -1 |  |
| UDWal20 | Dead | 1.41 | Linear Add |
| UDWal20 | Live | 1 |  |
| UDWal20 | Snow | 0.2 |  |
| UDWal20 | LPart | 1 |  |
| UDWal20 | EYAll-0.3EX | 1 |  |
| UDWal21 | Dead | 1.41 | Linear Add |
| UDWal21 | Live | 1 |  |
| UDWal21 | Snow | 0.2 |  |
| UDWal21 | LPart | 1 |  |
| UDWal21 | EYAll-0.3EX | -1 |  |
| UDWal22 | Dead | 0.69 | Linear Add |
| UDWal22 | EX | 1 |  |
| UDWal23 | Dead | 0.69 | Linear Add |
| UDWal23 | EX | -1 |  |
| UDWal24 | Dead | 0.69 | Linear Add |
| UDWal24 | EXAll | 1 |  |
| UDWal25 | Dead | 0.69 | Linear Add |
| UDWal25 | EXAll | -1 |  |
| UDWal26 | Dead | 0.69 | Linear Add |
| UDWal26 | EY | 1 |  |
| UDWal27 | Dead | 0.69 | Linear Add |
| UDWal27 | EY | -1 |  |
| UDWal28 | Dead | 0.69 | Linear Add |
| UDWal28 | EYAll | 1 |  |
| UDWal29 | Dead | 0.69 | Linear Add |
| UDWal29 | EYAll | -1 |  |
| UDWal30 | Dead | 0.69 | Linear Add |
| UDWal30 | EXAll+0.3EY | 1 |  |
| UDWal31 | Dead | 0.69 | Linear Add |
| UDWal31 | EXAll+0.3EY | -1 |  |
| UDWal32 | Dead | 0.69 | Linear Add |
| UDWal32 | EXAll-0.3EY | 1 |  |
| UDWal33 | Dead | 0.69 | Linear Add |
| UDWal33 | EXAll-0.3EY | -1 |  |
| UDWal34 | Dead | 0.69 | Linear Add |
| UDWal34 | EYAll+0.3EX | 1 |  |
| UDWal35 | Dead | 0.69 | Linear Add |
| UDWal35 | EYAll+0.3EX | -1 |  |
| UDWal36 | Dead | 0.69 | Linear Add |
| UDWal36 | EYAll-0.3EX | 1 |  |
| UDWal37 | Dead | 0.69 | Linear Add |
| UDWal37 | EYAll-0.3EX | -1 |  |

- در ترکیبات بار قاعده 30-100 نیز همانطورکه مشاهده میشود اعمال شده است.

مدلسازی دیوار برشی

مشخصات دیوار برشی های معمولی و بازشودار

اشکال هندسی دیوار برشی

اشکال مختلف دیوا برشی

در این پروژه دیوارهای برشی در قسمتهای گوشه بصورت L شکل بوده و در بقیه دیوارها بصورت مستطیلی بوده که در ادامه با توجه به توضیحات آنها را از نوع دمبلی شکل در نظر خواهیم گرفت و دیوارهای واقع در قاب B و قاب E از نوع دیوار برشی کوپله میباشد که طول تیر همبند در آنها برابر 8/1 متر میباشد.

محل دیوارهای برشی L شکل در پلان

محل دیوارهای برشی دمبلی شکل در پلان

محل دیوارهای برشی کوپله در پلان

ضخامت دیوار برشی

حداقل ضخامت دیوار برشی طبق مبحث نهم مقررات ملی ساختمان برابر 150 میلیمتر میباشد اما طبق توصیه مهندسین و منابع طراحی لرزهای سازههای بتنی بهتر است ضخامت دیوارهای برشی حداقل برابر 200 میلیمتر در نظر گرفته شود و با توجه به اینکه ضخامت دیوار برشی در این پروژه قرار است در همه دیوارها یکسان باشد به همین سبب ضخامت دیوارهای برشی برابر 200 میلیمتر در نظر گرفتهایم.

تعریف مقطع دیوار برشی در ایتبس

حداقل و حداکثر درصد هندسی آرماتور دیوارها

المان مرزی دیوار

- با توجه به توضیحات فوق برای در نظر گرفتن المان مرزی دیوارهای برشی ستونهای اطراف دیوار را جزئی از دیوار در نظر خواهیم گرفت به همین خاطر دیوارهای برشی بصورت مستطیلی نخواهند شد و با ادغام ستون داخل دیوار به شکل دمبلی شکل خواهند بود.

مش بندی دیوار برشی

المان های سطحی مدل شده در نرم افزار بایستی به ابعاد کوچکی تقسیم شوند تا نرم افزار بتواند به خوبی نیروها را انتقال داده و نتایج را به ما نشان دهند به همین جهت المان های سطحی بهتر است بصورت مربع هایی با اضلاع 5/0 در 5/0 متر یا 1 متر در 1 متر تقسیم بندی شوند.

مش بندی دیوارهای قاب A

مش بندی دیوارهای قاب B

در این قاب با توجه به اینکه دیوار برشی بصورت کوپه میباشد مش بندی بگونهای انجام گرفته که طول تیر همبند برابر 8/1 متر بوده و 3 مش افقی در آن خواهیم داشت و ارتفاع تیر کوپله نیز برابر 4/2 متر در نظر گرفته شده است که بدین ترتیب 4 مش قائم خواهیم داشت که در این قسمت ابعاد مش بندی 6/0 در 6/0 در نظر گرفته شده است.

مش بندی دیوارهای قاب C

مش بندی دیوارهای قاب D

مش بندی دیوارهای قاب E

در این قاب با توجه به اینکه دیوار برشی بصورت کوپه میباشد مش بندی بگونهای انجام گرفته که طول تیر همبند برابر 8/1 متر بوده و 3 مش افقی در آن خواهیم داشت و ارتفاع تیر کوپله نیز برابر 4/2 متر در نظر گرفته شده است که بدین ترتیب 4 مش قائم خواهیم داشت که در این قسمت ابعاد مش بندی 6/0 در 6/0 در نظر گرفته شده است.

مش بندی دیوارهای قاب F

مش بندی دیوارهای قاب 1

مش بندی دیوارهای قاب 6

برچسب دیوارها (نامگذاری دیوارها)

S1

P9

P10

P11

P12

S2

P13

P14

P15

P16

P8

P7

P6

P5

P4

P3

P2

P1

نام گذاری دیوارهای برشی

- در نام گذاری دیوارهای برشی P1 و P3 و P4 و P6 دیوارها بصورت L شکل در نظر گرفته شده و نام گذاری یکپارچه شدهاند تا نرم افزار در بخش طراحی کل این دیوارها را بصورت L شکل شناسایی و طراحی نماید.

تعریف Pier Labels در ایتبس برای دیوارهای برشی

تعریف Spandral Labels در ایتبس برای تیرهای همبند

برچسب دیوارهای P1

برچسب دیوارهای P2

برچسب دیوارهای P3

برچسب دیوارهای P4

برچسب دیوارهای P5

برچسب دیوارهای P6

برچسب دیوارهای P7

برچسب دیوارهای P8

برچسب دیوارهای قاب B

برچسب دیوارهای قاب E

ضرایب ترک خوردگی دیوارها

ضرایب ترک خوردگی دیوارها (در حالت ترک نخورده)

ضرایب ترک خوردگی ستونهای اطراف دیوار (در حالت ترک نخورده)

طول تیر همبند برابر 8/1 متر بوده و ارتفاع تیر همبند برابر 2/1 میباشد بنابراین خواهیم داشت:

Lnh=1.81.2=1.5

این مقدار بزرگتر اغز 4/1 بوده و کمتر از 2 میباشد برای این حالت میتوان درون یابی کرده و ضریب ترک خوردگی را بدست آورد اما در جهت اطمینان چون عدد بدست آمده نزدیک 4/1 میباشد از ضرایب 15/0 و 25/0 برای f11،f22 و f12 استفاده نموده و سختی خمشی خارج از صفحه را نیز 35/0 در نظر خواهیم گرفت.

ضرایب ترک خوردگی تیرهای همبند

کنترل ترک خوردگی دیوار

تنشهای کششی دیوار اگر از مدول گسیختگی دیوار بیشتر باشد دیوار ترک خورده و در غیر اینصورت ترک خورده محسوب خواهد شد.

کششی تنش≤fr                    نخورده ترک

کششی تنش>fr                    خورده ترک

fr=0.6λfc'=0.6×1×25=3Mpa

تنظیمات نمایش تنش دیوارها

تنشهای قاب A

تنشهای قاب B

تنشهای قاب C

تنشهای قاب D

تنشهای قاب E

تنشهای قاب 1

تنشهای قاب 6

مناطقی که در قسمت آبی پر رنگ قرار گرفتهاند نواحی هستند که تنش در آنها از مقدار گسیخگی بیشتر بوده و از 3 مگاپاسکال بیشتر هستند و ترک خورده محسوب میشود و ضرایب ترک خوردگی در این نواحی بایستی برابر 35/0 وارد شود.

3) تحلیل سازه

با محاسبه و تعیین بار های ثقلی و جانبی وارد بر سازه، سازه مورد نظر تحت بارهای مورد اشاره آنالیز می شود. برای تحلیل سازه از نرم افزار ETABS 2016 استفاده شده است.

2-3- کنترل نظم پیچشی ساختمان

| TABLE: Diaphragm Max/Avg Drifts |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Story | Load Case/Combo | Item | Max Drift | Avg Drift | Ratio |
| Story8 | EXAll 1 | Diaph D1 X | 0.002983 | 0.002983 | 1 |
| Story8 | EXAll 2 | Diaph D1 X | 0.003187 | 0.002983 | 1.068 |
| Story8 | EXAll 3 | Diaph D1 X | 0.003188 | 0.002983 | 1.069 |
| Story8 | EYAll 1 | Diaph D1 Y | 0.002878 | 0.002827 | 1.018 |
| Story8 | EYAll 2 | Diaph D1 Y | 0.002922 | 0.002823 | 1.035 |
| Story8 | EYAll 3 | Diaph D1 Y | 0.003033 | 0.002831 | 1.071 |
| Story7 | EXAll 1 | Diaph D1 X | 0.002995 | 0.002994 | 1 |
| Story7 | EXAll 2 | Diaph D1 X | 0.003199 | 0.002994 | 1.068 |
| Story7 | EXAll 3 | Diaph D1 X | 0.0032 | 0.002994 | 1.069 |
| Story7 | EYAll 1 | Diaph D1 Y | 0.002936 | 0.002885 | 1.018 |
| Story7 | EYAll 2 | Diaph D1 Y | 0.00298 | 0.00288 | 1.035 |
| Story7 | EYAll 3 | Diaph D1 Y | 0.003092 | 0.002889 | 1.07 |
| Story6 | EXAll 1 | Diaph D1 X | 0.00295 | 0.002949 | 1 |
| Story6 | EXAll 2 | Diaph D1 X | 0.003152 | 0.00295 | 1.069 |
| Story6 | EXAll 3 | Diaph D1 X | 0.003153 | 0.002949 | 1.069 |
| Story6 | EYAll 1 | Diaph D1 Y | 0.002939 | 0.002887 | 1.018 |
| Story6 | EYAll 2 | Diaph D1 Y | 0.00298 | 0.002882 | 1.034 |
| Story6 | EYAll 3 | Diaph D1 Y | 0.003093 | 0.002891 | 1.07 |
| Story5 | EXAll 1 | Diaph D1 X | 0.002833 | 0.002832 | 1 |
| Story5 | EXAll 2 | Diaph D1 X | 0.003027 | 0.002832 | 1.069 |
| Story5 | EXAll 3 | Diaph D1 X | 0.003028 | 0.002832 | 1.069 |
| Story5 | EYAll 1 | Diaph D1 Y | 0.002858 | 0.002805 | 1.019 |
| Story5 | EYAll 2 | Diaph D1 Y | 0.002891 | 0.0028 | 1.032 |
| Story5 | EYAll 3 | Diaph D1 Y | 0.003006 | 0.00281 | 1.07 |
| Story4 | EXAll 1 | Diaph D1 X | 0.002633 | 0.002632 | 1 |
| Story4 | EXAll 2 | Diaph D1 X | 0.002813 | 0.002632 | 1.069 |
| Story4 | EXAll 3 | Diaph D1 X | 0.002815 | 0.002632 | 1.069 |
| Story4 | EYAll 1 | Diaph D1 Y | 0.002672 | 0.002619 | 1.02 |
| Story4 | EYAll 2 | Diaph D1 Y | 0.002694 | 0.002614 | 1.031 |
| Story4 | EYAll 3 | Diaph D1 Y | 0.002811 | 0.002624 | 1.071 |
| Story3 | EXAll 1 | Diaph D1 X | 0.002224 | 0.002223 | 1 |
| Story3 | EXAll 2 | Diaph D1 X | 0.002376 | 0.002223 | 1.069 |
| Story3 | EXAll 3 | Diaph D1 X | 0.002378 | 0.002223 | 1.07 |
| Story3 | EYAll 1 | Diaph D1 Y | 0.002304 | 0.002249 | 1.024 |
| Story3 | EYAll 2 | Diaph D1 Y | 0.002301 | 0.002243 | 1.026 |
| Story3 | EYAll 3 | Diaph D1 Y | 0.002423 | 0.002255 | 1.075 |
| Story2 | EXAll 1 | Diaph D1 X | 0.00153 | 0.001529 | 1 |
| Story2 | EXAll 2 | Diaph D1 X | 0.001636 | 0.001529 | 1.07 |
| Story2 | EXAll 3 | Diaph D1 X | 0.001638 | 0.001529 | 1.071 |
| Story2 | EYAll 1 | Diaph D1 Y | 0.001654 | 0.00161 | 1.028 |
| Story2 | EYAll 2 | Diaph D1 Y | 0.001639 | 0.001605 | 1.022 |
| Story2 | EYAll 3 | Diaph D1 Y | 0.001739 | 0.001615 | 1.077 |
| Story1 | EXAll 1 | Diaph D1 X | 0.000654 | 0.000654 | 1.001 |
| Story1 | EXAll 2 | Diaph D1 X | 0.0007 | 0.000654 | 1.071 |
| Story1 | EXAll 3 | Diaph D1 X | 0.000701 | 0.000654 | 1.072 |
| Story1 | EYAll 1 | Diaph D1 Y | 0.000728 | 0.00071 | 1.025 |
| Story1 | EYAll 2 | Diaph D1 Y | 0.000725 | 0.000709 | 1.024 |
| Story1 | EYAll 3 | Diaph D1 Y | 0.000764 | 0.000712 | 1.073 |

بنا به بند فوق استاندارد 2800 ویرایش چهارم زمانیکه  جابجایی ماکزیمم به جابجایی میانگین اگر بزرگتر از 2/1 باشد سازه دارای نامنظمی زیاد پیچشی و زمانیکه دارای نسبت بزرگتر از 4/1 باشد سازه دارای نامنظمی شدید پیچشی خواهد بود که براازه ما مشاهده میشود تمامی نسبتها کوچکتر از 2/1 بوده و فاقد نامنظمی پیچشی میباشد.

3-3-کنترل تفییرمکان نسبی طبقات ( Drift )

پس از تعیین دوره تناوب تحلیلی نرم افزار، ضرایب C و K ( ضرایبی که بر اساس زمان تناوب تحلیلی بدست آمد ) برای نیرو های Ex Drift و EY Drift به نرم افزار معرفی می شود. پس از تحلیل سازه تغییر مکان های نسبی هر طبقه به صورت زیر محاسبه می گردد:

بعد از بدست آوردن مقادیر دریفت بایستی آنها را با مقادیر مجاز کنترل نمائیم که با توجه به اینکه سازه ما 8 طبقه میباشد بایستی مقادیر دریفت کوچکتر مساوی 0.02 تقسیم بر ضریب بزرگنمایی (Cd) باشند.

∆euh≤0.02Cd=0.024.5=0.004444

| TABLE: Diaphragm Max/Avg Drifts |  |  |  |  |
| --- | --- | --- | --- | --- |
| Story | Load Case/Combo | Item | Max Drift | Avg Drift |
| Story8 | EX | Diaph D1 X | 0.002752 | 0.002751 |
| Story8 | EY | Diaph D1 Y | 0.002655 | 0.002607 |
| Story7 | EX | Diaph D1 X | 0.002762 | 0.002761 |
| Story7 | EY | Diaph D1 Y | 0.002708 | 0.002661 |
| Story6 | EX | Diaph D1 X | 0.002721 | 0.00272 |
| Story6 | EY | Diaph D1 Y | 0.002711 | 0.002663 |
| Story5 | EX | Diaph D1 X | 0.002613 | 0.002612 |
| Story5 | EY | Diaph D1 Y | 0.002636 | 0.002587 |
| Story4 | EX | Diaph D1 X | 0.002429 | 0.002428 |
| Story4 | EY | Diaph D1 Y | 0.002465 | 0.002416 |
| Story3 | EX | Diaph D1 X | 0.002051 | 0.00205 |
| Story3 | EY | Diaph D1 Y | 0.002125 | 0.002074 |
| Story2 | EX | Diaph D1 X | 0.001411 | 0.001411 |
| Story2 | EY | Diaph D1 Y | 0.001526 | 0.001485 |
| Story1 | EX | Diaph D1 X | 0.000603 | 0.000603 |
| Story1 | EY | Diaph D1 Y | 0.000672 | 0.000655 |

نسبت ∆euh را که در بالا محاسبه کردهایم بایستی برای تمامی طبقات کوچکتر از 004444/0 باشند که مشاهده میشود برای هر دو جهت تمامی مقادیر از این مقدار کوچکتر بوده و نشان میدهد که از لحاظ دریفت در هر دو جهت مشکلی نخواهیم داشت.

4)طراحی سازه

طراحی تیرها و ستون های سازه با معرفی مقاطع به نرم افزار ETABS 2016 و کنترل کفایت آن ها پس از چند مرحله تحلیل و طراحی انجام می پذیرد. طراحی اسکلت سازه براساس آیین نامه ACI 318-14 و کنترل ضوابط شکل پذیری با استفاده از مبحث نهم مقررات ملی انجام شده است.

1-4- مقاطع ستونها و تیرها

| TABLE: Frame Assignments - Sections |  |  |  |  |
| --- | --- | --- | --- | --- |
| Story | Label | Design Type | Design Procedure | Design Section |
| Story8 | C1 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C2 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C3 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C4 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C5 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C6 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C7 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C8 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C9 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C10 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C11 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C12 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C13 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C14 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C15 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C16 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C17 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C18 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C19 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C20 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C21 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C22 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C23 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C24 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C25 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C26 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C27 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C28 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C29 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C30 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C31 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C32 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C33 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C34 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C35 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story8 | C36 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C1 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C2 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C3 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C4 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C5 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C6 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C7 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C8 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C9 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C10 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C11 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C12 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C13 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C14 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C15 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C16 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C17 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C18 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C19 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C20 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C21 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C22 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C23 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C24 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C25 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C26 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C27 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C28 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C29 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C30 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C31 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C32 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C33 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C34 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C35 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story7 | C36 | Column | Concrete Frame Design | CL45X45-12F16 |
| Story6 | C1 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C2 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C3 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C4 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C5 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C6 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C7 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C8 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C9 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C10 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C11 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C12 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C13 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C14 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C15 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C16 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C17 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C18 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C19 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C20 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C21 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C22 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C23 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C24 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C25 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C26 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C27 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C28 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C29 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C30 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C31 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C32 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C33 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C34 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C35 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story6 | C36 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C1 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C2 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C3 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C4 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C5 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C6 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C7 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C8 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C9 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C10 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C11 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C12 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C13 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C14 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C15 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C16 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C17 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C18 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C19 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C20 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C21 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C22 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C23 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C24 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C25 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C26 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C27 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C28 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C29 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C30 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C31 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C32 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C33 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C34 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C35 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story5 | C36 | Column | Concrete Frame Design | CL50X50-16F20 |
| Story4 | C1 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C2 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C3 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C4 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C5 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C6 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C7 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C8 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C9 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C10 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C11 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C12 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C13 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C14 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C15 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C16 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C17 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C18 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C19 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C20 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C21 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C22 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C23 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C24 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C25 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C26 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C27 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C28 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C29 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C30 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C31 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C32 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C33 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C34 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C35 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story4 | C36 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C1 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C2 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C3 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C4 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C5 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C6 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C7 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C8 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C9 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C10 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C11 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C12 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C13 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C14 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C15 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C16 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C17 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C18 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C19 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C20 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C21 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C22 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C23 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C24 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C25 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C26 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C27 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C28 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C29 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C30 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C31 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C32 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C33 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C34 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C35 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story3 | C36 | Column | Concrete Frame Design | CL55X55-20F20 |
| Story2 | C1 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C2 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C3 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C4 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C5 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C6 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C7 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C8 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C9 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C10 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C11 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C12 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C13 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C14 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C15 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C16 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C17 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C18 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C19 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C20 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C21 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C22 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C23 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C24 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C25 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C26 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C27 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C28 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C29 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C30 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C31 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C32 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C33 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C34 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C35 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story2 | C36 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C1 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C2 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C3 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C4 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C5 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C6 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C7 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C8 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C9 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C10 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C11 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C12 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C13 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C14 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C15 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C16 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C17 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C18 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C19 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C20 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C21 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C22 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C23 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C24 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C25 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C26 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C27 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C28 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C29 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C30 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C31 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C32 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C33 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C34 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C35 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story1 | C36 | Column | Concrete Frame Design | CL60X60-20F20 |
| Story8 | B1 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B2 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B3 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B4 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B5 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B6 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B7 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B8 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B9 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B10 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B11 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B12 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B13 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B14 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B15 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B16 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B17 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B18 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B19 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B20 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B21 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B22 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B23 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B24 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B25 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B26 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B27 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B28 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B29 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B30 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B31 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B32 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B33 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B34 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B35 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B36 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B37 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B38 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B39 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B40 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B41 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B42 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B43 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B44 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B45 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B46 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B47 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B48 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B49 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B50 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B51 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B52 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B53 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B54 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B55 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B56 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B57 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B58 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B59 | Beam | Concrete Frame Design | B45X35 |
| Story8 | B60 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B1 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B2 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B3 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B4 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B5 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B6 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B7 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B8 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B9 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B10 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B11 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B12 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B13 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B14 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B15 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B16 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B17 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B18 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B19 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B20 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B21 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B22 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B23 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B24 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B25 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B26 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B27 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B28 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B29 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B30 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B31 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B32 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B33 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B34 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B35 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B36 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B37 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B38 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B39 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B40 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B41 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B42 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B43 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B44 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B45 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B46 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B47 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B48 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B49 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B50 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B51 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B52 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B53 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B54 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B55 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B56 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B57 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B58 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B59 | Beam | Concrete Frame Design | B45X35 |
| Story7 | B60 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B1 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B2 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B3 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B4 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B5 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B6 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B7 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B8 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B9 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B10 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B11 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B12 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B13 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B14 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B15 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B16 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B17 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B18 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B19 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B20 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B21 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B22 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B23 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B24 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B25 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B26 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B27 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B28 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B29 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B30 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B31 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B32 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B33 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B34 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B35 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B36 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B37 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B38 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B39 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B40 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B41 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B42 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B43 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B44 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B45 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B46 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B47 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B48 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B49 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B50 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B51 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B52 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B53 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B54 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B55 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B56 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B57 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B58 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B59 | Beam | Concrete Frame Design | B45X35 |
| Story6 | B60 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B1 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B2 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B3 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B4 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B5 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B6 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B7 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B8 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B9 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B10 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B11 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B12 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B13 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B14 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B15 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B16 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B17 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B18 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B19 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B20 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B21 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B22 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B23 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B24 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B25 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B26 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B27 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B28 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B29 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B30 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B31 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B32 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B33 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B34 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B35 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B36 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B37 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B38 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B39 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B40 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B41 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B42 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B43 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B44 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B45 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B46 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B47 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B48 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B49 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B50 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B51 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B52 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B53 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B54 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B55 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B56 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B57 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B58 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B59 | Beam | Concrete Frame Design | B45X35 |
| Story5 | B60 | Beam | Concrete Frame Design | B45X35 |
| Story4 | B1 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B2 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B3 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B4 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B5 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B6 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B7 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B8 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B9 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B10 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B11 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B12 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B13 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B14 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B15 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B16 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B17 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B18 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B19 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B20 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B21 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B22 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B23 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B24 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B25 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B26 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B27 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B28 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B29 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B30 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B31 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B32 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B33 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B34 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B35 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B36 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B37 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B38 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B39 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B40 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B41 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B42 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B43 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B44 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B45 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B46 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B47 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B48 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B49 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B50 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B51 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B52 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B53 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B54 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B55 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B56 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B57 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B58 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B59 | Beam | Concrete Frame Design | B55X40 |
| Story4 | B60 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B1 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B2 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B3 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B4 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B5 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B6 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B7 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B8 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B9 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B10 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B11 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B12 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B13 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B14 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B15 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B16 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B17 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B18 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B19 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B20 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B21 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B22 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B23 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B24 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B25 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B26 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B27 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B28 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B29 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B30 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B31 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B32 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B33 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B34 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B35 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B36 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B37 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B38 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B39 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B40 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B41 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B42 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B43 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B44 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B45 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B46 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B47 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B48 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B49 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B50 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B51 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B52 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B53 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B54 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B55 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B56 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B57 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B58 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B59 | Beam | Concrete Frame Design | B55X40 |
| Story3 | B60 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B1 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B2 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B3 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B4 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B5 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B6 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B7 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B8 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B9 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B10 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B11 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B12 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B13 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B14 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B15 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B16 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B17 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B18 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B19 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B20 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B21 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B22 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B23 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B24 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B25 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B26 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B27 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B28 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B29 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B30 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B31 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B32 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B33 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B34 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B35 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B36 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B37 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B38 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B39 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B40 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B41 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B42 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B43 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B44 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B45 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B46 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B47 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B48 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B49 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B50 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B51 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B52 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B53 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B54 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B55 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B56 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B57 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B58 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B59 | Beam | Concrete Frame Design | B55X40 |
| Story2 | B60 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B1 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B2 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B3 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B4 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B5 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B6 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B7 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B8 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B9 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B10 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B11 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B12 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B13 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B14 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B15 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B16 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B17 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B18 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B19 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B20 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B21 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B22 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B23 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B24 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B25 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B26 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B27 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B28 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B29 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B30 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B31 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B32 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B33 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B34 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B35 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B36 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B37 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B38 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B39 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B40 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B41 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B42 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B43 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B44 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B45 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B46 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B47 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B48 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B49 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B50 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B51 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B52 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B53 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B54 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B55 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B56 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B57 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B58 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B59 | Beam | Concrete Frame Design | B55X40 |
| Story1 | B60 | Beam | Concrete Frame Design | B55X40 |

1-5-نسبت تنش ها در ستون های قاب ها

1-1-5-قاب A

2-1-5-قاب B

3-1-5-قاب C

4-1-5-قاب D

5-1-5-قاب E

5-1-5-قاب F

- ستونهایی که نسبت نیرو به ظرفیت آنها بیشتر از 1 میباشد ستونهای اطراف دیوار برشی میباشند که چون آنها جزئی از دیوار برشی بعنوان المان مرزی در نظر گرفته شدهاند به همین جهت جوابگویی آنها لازم نبوده و نسبت نیرو به ظرفیت آنها مهم نمیباشد.

طراحی دیوارهای برشی

دیوارها از هر 2 طبقه تیپ بندی خواهند شد که در ادامه به طراحی آنها خواهیم پرداخت.

برای طراحی دیوارهای برشی بهتر است آرماتورهای قائم را در دو انتهای دیوار متمرکز نمائیم (یعنی از حداکثر آرماتور در دو انتها استفاده نمائیم) و در وسط دیوار از آرماتور حداقل تا حد امکان استفاده نمائیم.

- دیوارهای برشی دمبلی شکل و L شکل

- دیوارهای طبقه اول و دوم

ρ=Asbh→0.02=As60×60→As=72 cm2

USE 20∅20 →As.Prov=62.8 cm2

برای آرماتورهای وسط تیر نیز خواهیم داشت:

2∅14@200 mm→ρ=2×π×1.42420×20=0.0077

- دیوارهای P2، P5، P7 و P8 جوابگو بوده اما بقیه مقاطع دیوار جوابگو نیست و برای این منظور فاصله آرماتورها در وسط دیوار را کم میکنیم.

2∅14@100 mm→ρ=2×π×1.42410×20=0.0154

- دیوارهای P3، P6 جوابگو نبوده که نسبت نیرو به ظرفیت بالایی دارند و در این مقاطع آرماتورهای وسط دیوار را نمره 16 انتخاب میکنیم اما مقاطع P1 و P4 و P6 با اختلاف خیلی کمی جوابگو نیستند به همین منظور در آنها فواصل آرماتورها را 90 سانتیمتر در نظر میگیریم.

2∅16@90 mm→ρ=2×π×1.62410×20=0.021

2∅14@90 mm→ρ=2×π×1.4249×20=0.0171

در این حالت مقاطع جوابگو هستند و برخی مقاطع با اختلاف بسیار اندکی جوابگو نیستند که به منظور تیپ بندی همان مقاطع ماندهاند.

همین کار را برای بقیه طبقات نیز انجام داده و مقاطع دیوارها را بدست میآوریم که در ادمه شکل مقاطع آورده شدهاند.

P1-Story1&2

P2&P5-Story1&2

P3-Story1&2

P4-Story1&2

P6-Story1&2

P7&P8-Story1&2

- آرماتورگذاری دیوارهای برشی

- طراحی دیوارهای برشی کوپله و تیرهای همبند

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story1 | S1 | 6000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 986 | 0.41 | UDWal21 | -368722039 |
| Right | 986 | 0.41 | UDWal20 | -368636358 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 883 | 0.37 | UDWal36 | 331518588 |
| Right | 883 | 0.37 | UDWal37 | 331522815 |

مشخصات تیر همبند (S1) – طبقه اول

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story2 | S1 | 6000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1513 | 0.63 | UDWal21 | -553652066 |
| Right | 1513 | 0.63 | UDWal20 | -553488514 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1418 | 0.59 | UDWal36 | 520839191 |
| Right | 1418 | 0.59 | UDWal37 | 520882213 |

مشخصات تیر همبند (S1) – طبقه دوم

As=1513 mm2

USE 5∅20 Top & Bot

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story3 | S1 | 6000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1748 | 0.73 | UDWal21 | -633534555 |
| Right | 1748 | 0.73 | UDWal20 | -633355758 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1654 | 0.69 | UDWal36 | 601811052 |
| Right | 1654 | 0.69 | UDWal37 | 601853253 |

مشخصات تیر همبند (S1) – طبقه سوم

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story4 | S1 | 6000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1656 | 0.69 | UDWal21 | -602424350 |
| Right | 1656 | 0.69 | UDWal20 | -602283937 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1576 | 0.66 | UDWal36 | 575048981 |
| Right | 1576 | 0.66 | UDWal37 | 575077274 |

مشخصات تیر همبند (S1) – طبقه چهارم

As=1748 mm2

USE 6∅20 Top & Bot

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story5 | S1 | 6000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1485 | 0.62 | UDWal21 | -544067183 |
| Right | 1485 | 0.62 | UDWal20 | -544075068 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1417 | 0.59 | UDWal36 | 520512589 |
| Right | 1417 | 0.59 | UDWal37 | 520497431 |

مشخصات تیر همبند (S1) – طبقه پنجم

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story6 | S1 | 6000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1251 | 0.52 | UDWal21 | -462666771 |
| Right | 1251 | 0.52 | UDWal20 | -462712195 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1194 | 0.5 | UDWal36 | 442799268 |
| Right | 1194 | 0.5 | UDWal37 | 442757801 |

مشخصات تیر همبند (S1) – طبقه ششم

As=1485 mm2

USE 5∅20 Top & Bot

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story7 | S1 | 6000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1035 | 0.43 | UDWal21 | -386252398 |
| Right | 1035 | 0.43 | UDWal20 | -386288959 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 985 | 0.41 | UDWal36 | 368471531 |
| Right | 985 | 0.41 | UDWal37 | 368438539 |

مشخصات تیر همبند (S1) – طبقه هفتم

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story8 | S1 | 6000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 797 | 0.33 | UDWal21 | -300398001 |
| Right | 798 | 0.33 | UDWal20 | -300452552 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 755 | 0.31 | UDWal36 | 285012177 |
| Right | 755 | 0.31 | UDWal37 | 284958637 |

مشخصات تیر همبند (S1) – طبقه هشتم

As=1035 mm2

USE 4∅20 Top & Bot

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story1 | S2 | 24000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 948 | 0.4 | UDWal19 | -355073363 |
| Right | 948 | 0.39 | UDWal18 | -354977148 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 845 | 0.35 | UDWal34 | 317865560 |
| Right | 845 | 0.35 | UDWal35 | 317867112 |

مشخصات تیر همبند (S2) – طبقه اول

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story2 | S2 | 24000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1446 | 0.6 | UDWal19 | -530431287 |
| Right | 1445 | 0.6 | UDWal18 | -530249743 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1351 | 0.56 | UDWal34 | 497608550 |
| Right | 1351 | 0.56 | UDWal35 | 497652607 |

مشخصات تیر همبند (S2) – طبقه دوم

As=1446 mm2

USE 5∅20 Top & Bot

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story3 | S2 | 24000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1683 | 0.7 | UDWal19 | -611526878 |
| Right | 1682 | 0.7 | UDWal18 | -611332025 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1590 | 0.66 | UDWal34 | 579794803 |
| Right | 1590 | 0.66 | UDWal35 | 579837650 |

مشخصات تیر همبند (S2) – طبقه سوم

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story4 | S2 | 24000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1611 | 0.67 | UDWal19 | -586955884 |
| Right | 1610 | 0.67 | UDWal18 | -586807967 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1530 | 0.64 | UDWal34 | 559577590 |
| Right | 1530 | 0.64 | UDWal35 | 559604183 |

مشخصات تیر همبند (S2) – طبقه چهارم

As=1683 mm2

USE 6∅20 Top & Bot

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story5 | S2 | 24000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1451 | 0.6 | UDWal19 | -532316624 |
| Right | 1451 | 0.6 | UDWal18 | -532321359 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1383 | 0.58 | UDWal34 | 508760558 |
| Right | 1383 | 0.58 | UDWal35 | 508745148 |

مشخصات تیر همبند (S2) – طبقه پنجم

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story6 | S2 | 24000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1225 | 0.51 | UDWal19 | -453684175 |
| Right | 1225 | 0.51 | UDWal18 | -453728179 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1169 | 0.49 | UDWal34 | 433816805 |
| Right | 1169 | 0.49 | UDWal35 | 433773639 |

مشخصات تیر همبند (S2) – طبقه ششم

As=1451 mm2

USE 5∅20 Top & Bot

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story7 | S2 | 24000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1015 | 0.42 | UDWal19 | -378944970 |
| Right | 1015 | 0.42 | UDWal18 | -378981827 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 965 | 0.4 | UDWal34 | 361165385 |
| Right | 965 | 0.4 | UDWal35 | 361130064 |

مشخصات تیر همبند (S2) – طبقه هفتم

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story8 | S2 | 24000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 781 | 0.33 | UDWal19 | -294532936 |
| Right | 781 | 0.33 | UDWal18 | -294587300 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 745 | 0.31 | UDWal34 | 279148238 |
| Right | 745 | 0.31 | UDWal35 | 279092422 |

مشخصات تیر همبند (S2) – طبقه هشتم

As=1015 mm2

USE 4∅20 Top & Bot

- تیرهای همبند با ضخامت 20 سانتیمتری جوابگو نبوده و مقدار آرماتورهای قطری توسط نرم افزار گزارش داده نمیشود به همین سبب ضخامت تیرهای همبند را به 40 سانتیمتر افزایش میدهیم تا در برابر نیروی برشی جوابگو باشند.

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story1 | S1 | 6000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1289 | 0.27 | UDWal21 | -368722039 |
| Right | 1288 | 0.27 | UDWal20 | -368636358 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1156 | 0.24 | UDWal36 | 331518588 |
| Right | 1156 | 0.24 | UDWal37 | 331522815 |

Spandrel Shear Design

| Station Location | Avert mm²/mm | Ahoriz mm²/mm | ShearCombo | Vu N | ϕVc N | ϕVs N | ϕVn N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 1.86 | 1 | UDWal36 | 644970.57 | 163290.39 | 481680.18 | 644970.57 |
| Right | 1.86 | 1 | UDWal37 | 644992.1 | 163294.61 | 481697.5 | 644992.1 |

Spandrel Shear Design—Diagonal Reinforcement

| Station Location | Adiag mm² | Shear Combo | Vu N | VuLimit N | L/H Ratio | Seismic Design | Diag Reinf Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 2266 | UDWal21 | 725128.17 | 759245.52 | 1.5 | Yes | No |
| Right | 2266 | UDWal20 | 724969.11 | 759245.52 | 1.5 | Yes | No |

مشخصات تیر همبند (S1) – طبقه اول

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story2 | S1 | 6000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1489 | 0.31 | UDWal21 | -553652066 |
| Right | 1489 | 0.31 | UDWal20 | -553488514 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1489 | 0.31 | UDWal36 | 520839191 |
| Right | 1489 | 0.31 | UDWal37 | 520882213 |

Spandrel Shear Design

| Station Location | Avert mm²/mm | Ahoriz mm²/mm | ShearCombo | Vu N | ϕVc N | ϕVs N | ϕVn N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 3.4 | 1 | UDWal36 | 1008494.54 | 127864.82 | 880629.73 | 1008494.54 |
| Right | 3.4 | 1 | UDWal37 | 1008591.34 | 127861.22 | 880730.12 | 1008591.34 |

Spandrel Shear Design—Diagonal Reinforcement

| Station Location | Adiag mm² | Shear Combo | Vu N | VuLimit N | L/H Ratio | Seismic Design | Diag Reinf Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 3389 | UDWal21 | 1084357.42 | 759245.52 | 1.5 | Yes | Yes |
| Right | 3388 | UDWal20 | 1084040.39 | 759245.52 | 1.5 | Yes | Yes |

مشخصات تیر همبند (S1) – طبقه دوم

As=1489 mm2→USE 5∅20 Top & Bot             طولی

As=3389 mm2→USE 7∅25 Left & Right             قطری

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story3 | S1 | 6000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1685 | 0.35 | UDWal21 | -633534555 |
| Right | 1684 | 0.35 | UDWal20 | -633355758 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1598 | 0.33 | UDWal36 | 601811052 |
| Right | 1598 | 0.33 | UDWal37 | 601853253 |

Spandrel Shear Design

| Station Location | Avert mm²/mm | Ahoriz mm²/mm | ShearCombo | Vu N | ϕVc N | ϕVs N | ϕVn N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 2.9 | 1 | UDWal36 | 1165759.92 | 113384.38 | 1052375.54 | 1165759.92 |
| Right | 2.9 | 1 | UDWal37 | 1165869.15 | 113384.3 | 1052484.85 | 1165869.15 |

Spandrel Shear Design—Diagonal Reinforcement

| Station Location | Adiag mm² | Shear Combo | Vu N | VuLimit N | L/H Ratio | Seismic Design | Diag Reinf Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 3689 | UDWal37 | 1212549.95 | 759245.52 | 1.5 | Yes | Yes |
| Right | 3689 | UDWal36 | 1212363.37 | 759245.52 | 1.5 | Yes | Yes |

مشخصات تیر همبند (S1) – طبقه سوم

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story4 | S1 | 6000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1599 | 0.33 | UDWal21 | -602424350 |
| Right | 1599 | 0.33 | UDWal20 | -602283937 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1524 | 0.32 | UDWal36 | 575048981 |
| Right | 1524 | 0.32 | UDWal37 | 575077274 |

Spandrel Shear Design

| Station Location | Avert mm²/mm | Ahoriz mm²/mm | ShearCombo | Vu N | ϕVc N | ϕVs N | ϕVn N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 2.86 | 1 | UDWal10 | 1053828.41 | 124334.17 | 929494.24 | 1053828.41 |
| Right | 2.86 | 1 | UDWal20 | 1177258.12 | 254920.76 | 922337.37 | 1177258.12 |

Spandrel Shear Design—Diagonal Reinforcement

| Station Location | Adiag mm² | Shear Combo | Vu N | VuLimit N | L/H Ratio | Seismic Design | Diag Reinf Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 3774 | UDWal21 | 1177520.52 | 759245.52 | 1.5 | Yes | Yes |
| Right | 3774 | UDWal20 | 1177258.12 | 759245.52 | 1.5 | Yes | Yes |

مشخصات تیر همبند (S1) – طبقه چهارم

As=1685 mm2→USE 6∅20 Top & Bot             طولی

As=3774 mm2→USE 7∅25 Left & Right             قطری

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story5 | S2 | 24000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1489 | 0.31 | UDWal19 | -532316624 |
| Right | 1489 | 0.31 | UDWal18 | -532321359 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1489 | 0.31 | UDWal34 | 508760558 |
| Right | 1489 | 0.31 | UDWal35 | 508745148 |

Spandrel Shear Design

| Station Location | Avert mm²/mm | Ahoriz mm²/mm | ShearCombo | Vu N | ϕVc N | ϕVs N | ϕVn N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 3.3 | 1 | UDWal34 | 986392.67 | 131862.23 | 854530.44 | 986392.67 |
| Right | 3.3 | 1 | UDWal35 | 986370.3 | 131866.65 | 854503.65 | 986370.3 |

Spandrel Shear Design—Diagonal Reinforcement

| Station Location | Adiag mm² | Shear Combo | Vu N | VuLimit N | L/H Ratio | Seismic Design | Diag Reinf Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 3255 | UDWal19 | 1041484.6 | 759245.52 | 1.5 | Yes | Yes |
| Right | 3255 | UDWal18 | 1041485.94 | 759245.52 | 1.5 | Yes | Yes |

مشخصات تیر همبند (S1) – طبقه پنجم

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story6 | S1 | 6000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1489 | 0.31 | UDWal21 | -462666771 |
| Right | 1489 | 0.31 | UDWal20 | -462712195 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1489 | 0.31 | UDWal36 | 442799268 |
| Right | 1489 | 0.31 | UDWal37 | 442757801 |

Spandrel Shear Design

| Station Location | Avert mm²/mm | Ahoriz mm²/mm | ShearCombo | Vu N | ϕVc N | ϕVs N | ϕVn N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 2.75 | 1 | UDWal36 | 856053.26 | 144246.28 | 711806.98 | 856053.26 |
| Right | 2.75 | 1 | UDWal37 | 855991.71 | 144258.39 | 711733.32 | 855991.71 |

Spandrel Shear Design—Diagonal Reinforcement

| Station Location | Adiag mm² | Shear Combo | Vu N | VuLimit N | L/H Ratio | Seismic Design | Diag Reinf Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 2835 | UDWal21 | 907134.77 | 759245.52 | 1.5 | Yes | Yes |
| Right | 2835 | UDWal20 | 907225.99 | 759245.52 | 1.5 | Yes | Yes |

مشخصات تیر همبند (S1) – طبقه ششم

As=1489 mm2→USE 5∅20 Top & Bot             طولی

As=3255 mm2→USE 7∅25 Left & Right             قطری

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story7 | S1 | 6000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1351 | 0.28 | UDWal21 | -386252398 |
| Right | 1351 | 0.28 | UDWal20 | -386288959 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1288 | 0.27 | UDWal36 | 368471531 |
| Right | 1288 | 0.27 | UDWal37 | 368438539 |

Spandrel Shear Design

| Station Location | Avert mm²/mm | Ahoriz mm²/mm | ShearCombo | Vu N | ϕVc N | ϕVs N | ϕVn N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 2.13 | 1 | UDWal36 | 710723.16 | 158300.88 | 552422.28 | 710723.16 |
| Right | 2.13 | 1 | UDWal37 | 710671.36 | 158309.33 | 552362.02 | 710671.36 |

Spandrel Shear Design—Diagonal Reinforcement

| Station Location | Adiag mm² | Shear Combo | Vu N | VuLimit N | L/H Ratio | Seismic Design | Diag Reinf Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 2369 | UDWal21 | 758138.13 | 759245.52 | 1.5 | Yes | No |
| Right | 2369 | UDWal20 | 758206.57 | 759245.52 | 1.5 | Yes | No |

مشخصات تیر همبند (S1) – طبقه هفتم

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story8 | S1 | 6000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1046 | 0.22 | UDWal21 | -300398001 |
| Right | 1046 | 0.22 | UDWal20 | -300452552 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 992 | 0.21 | UDWal36 | 285012177 |
| Right | 992 | 0.21 | UDWal37 | 284958637 |

Spandrel Shear Design

| Station Location | Avert mm²/mm | Ahoriz mm²/mm | ShearCombo | Vu N | ϕVc N | ϕVs N | ϕVn N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 1.45 | 1 | UDWal36 | 550068.3 | 175065.19 | 375003.12 | 550068.3 |
| Right | 1.45 | 1 | UDWal37 | 550001.4 | 175085.17 | 374916.23 | 550001.4 |

Spandrel Shear Design—Diagonal Reinforcement

| Station Location | Adiag mm² | Shear Combo | Vu N | VuLimit N | L/H Ratio | Seismic Design | Diag Reinf Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 1851 | UDWal21 | 592319.5 | 759245.52 | 1.5 | Yes | No |
| Right | 1851 | UDWal20 | 592458.19 | 759245.52 | 1.5 | Yes | No |

مشخصات تیر همبند (S1) – طبقه هشتم

As=1355 mm2→USE 5∅20 Top & Bot             طولی

As=2369 mm2→USE 5∅25 Left & Right             قطری

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story1 | S2 | 24000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1240 | 0.26 | UDWal19 | -355073363 |
| Right | 1240 | 0.26 | UDWal18 | -354977148 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1108 | 0.23 | UDWal34 | 317865560 |
| Right | 1108 | 0.23 | UDWal35 | 317867112 |

Spandrel Shear Design

| Station Location | Avert mm²/mm | Ahoriz mm²/mm | ShearCombo | Vu N | ϕVc N | ϕVs N | ϕVn N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 1.75 | 1 | UDWal19 | 698650.24 | 245010.92 | 453639.32 | 698650.24 |
| Right | 1.75 | 1 | UDWal18 | 698474.34 | 245002.16 | 453472.19 | 698474.34 |

Spandrel Shear Design—Diagonal Reinforcement

| Station Location | Adiag mm² | Shear Combo | Vu N | VuLimit N | L/H Ratio | Seismic Design | Diag Reinf Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 2183 | UDWal19 | 698650.24 | 759245.52 | 1.5 | Yes | No |
| Right | 2183 | UDWal18 | 698474.34 | 759245.52 | 1.5 | Yes | No |

مشخصات تیر همبند (S2) – طبقه اول

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story2 | S2 | 24000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1489 | 0.31 | UDWal19 | -530431287 |
| Right | 1489 | 0.31 | UDWal18 | -530249743 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1489 | 0.31 | UDWal34 | 497608550 |
| Right | 1489 | 0.31 | UDWal35 | 497652607 |

Spandrel Shear Design

| Station Location | Avert mm²/mm | Ahoriz mm²/mm | ShearCombo | Vu N | ϕVc N | ϕVs N | ϕVn N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 3.21 | 1 | UDWal34 | 963429.18 | 132314.3 | 831114.88 | 963429.18 |
| Right | 3.21 | 1 | UDWal35 | 963531.87 | 132312.04 | 831219.83 | 963531.87 |

Spandrel Shear Design—Diagonal Reinforcement

| Station Location | Adiag mm² | Shear Combo | Vu N | VuLimit N | L/H Ratio | Seismic Design | Diag Reinf Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 3248 | UDWal19 | 1039310.87 | 759245.52 | 1.5 | Yes | Yes |
| Right | 3247 | UDWal18 | 1038962.96 | 759245.52 | 1.5 | Yes | Yes |

مشخصات تیر همبند (S2) – طبقه دوم

As=1489 mm2→USE 5∅20 Top & Bot             طولی

As=3248 mm2→USE 7∅25 Left & Right             قطری

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story3 | S2 | 24000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1624 | 0.34 | UDWal19 | -611526878 |
| Right | 1624 | 0.34 | UDWal18 | -611332025 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1537 | 0.32 | UDWal34 | 579794803 |
| Right | 1537 | 0.32 | UDWal35 | 579837650 |

Spandrel Shear Design

| Station Location | Avert mm²/mm | Ahoriz mm²/mm | ShearCombo | Vu N | ϕVc N | ϕVs N | ϕVn N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 3.02 | 1 | UDWal10 | 1067948.36 | 123449.67 | 944498.69 | 1067948.36 |
| Right | 3.01 | 1 | UDWal18 | 1193852.93 | 255914.27 | 937938.66 | 1193852.93 |

Spandrel Shear Design—Diagonal Reinforcement

| Station Location | Adiag mm² | Shear Combo | Vu N | VuLimit N | L/H Ratio | Seismic Design | Diag Reinf Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 3364 | UDWal19 | 1194220.85 | 759245.52 | 1.5 | Yes | Yes |
| Right | 3366 | UDWal18 | 1193852.93 | 759245.52 | 1.5 | Yes | Yes |

مشخصات تیر همبند (S2) – طبقه سوم

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story4 | S2 | 24000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1557 | 0.32 | UDWal19 | -586955884 |
| Right | 1556 | 0.32 | UDWal18 | -586807967 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1489 | 0.31 | UDWal34 | 559577590 |
| Right | 1489 | 0.31 | UDWal35 | 559604183 |

Spandrel Shear Design

| Station Location | Avert mm²/mm | Ahoriz mm²/mm | ShearCombo | Vu N | ϕVc N | ϕVs N | ϕVn N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 2.98 | 1 | UDWal12 | 1058523.65 | 123856.57 | 934667.07 | 1058523.65 |
| Right | 2.96 | 1 | UDWal11 | 1028207.84 | 126827.51 | 901380.32 | 1028207.84 |

Spandrel Shear Design—Diagonal Reinforcement

| Station Location | Adiag mm² | Shear Combo | Vu N | VuLimit N | L/H Ratio | Seismic Design | Diag Reinf Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 3146 | UDWal19 | 1147516.3 | 759245.52 | 1.5 | Yes | Yes |
| Right | 3150 | UDWal18 | 1147243.1 | 759245.52 | 1.5 | Yes | Yes |

مشخصات تیر همبند (S2) – طبقه چهارم

As=1628 mm2→USE 6∅20 Top & Bot             طولی

As=3366 mm2→USE 7∅25 Left & Right             قطری

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story5 | S2 | 24000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1489 | 0.31 | UDWal19 | -532316624 |
| Right | 1489 | 0.31 | UDWal18 | -532321359 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1489 | 0.31 | UDWal34 | 508760558 |
| Right | 1489 | 0.31 | UDWal35 | 508745148 |

Spandrel Shear Design

| Station Location | Avert mm²/mm | Ahoriz mm²/mm | ShearCombo | Vu N | ϕVc N | ϕVs N | ϕVn N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 3.3 | 1 | UDWal34 | 986392.67 | 131862.23 | 854530.44 | 986392.67 |
| Right | 3.3 | 1 | UDWal35 | 986370.3 | 131866.65 | 854503.65 | 986370.3 |

Spandrel Shear Design—Diagonal Reinforcement

| Station Location | Adiag mm² | Shear Combo | Vu N | VuLimit N | L/H Ratio | Seismic Design | Diag Reinf Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 3255 | UDWal19 | 1041484.6 | 759245.52 | 1.5 | Yes | Yes |
| Right | 3255 | UDWal18 | 1041485.94 | 759245.52 | 1.5 | Yes | Yes |

مشخصات تیر همبند (S2) – طبقه پنجم

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story6 | S2 | 24000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1489 | 0.31 | UDWal19 | -453684175 |
| Right | 1489 | 0.31 | UDWal18 | -453728179 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1489 | 0.31 | UDWal34 | 433816805 |
| Right | 1489 | 0.31 | UDWal35 | 433773639 |

Spandrel Shear Design

| Station Location | Avert mm²/mm | Ahoriz mm²/mm | ShearCombo | Vu N | ϕVc N | ϕVs N | ϕVn N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 2.67 | 1 | UDWal34 | 838604.51 | 145939.8 | 692664.71 | 838604.51 |
| Right | 2.67 | 1 | UDWal35 | 838541.52 | 145952.87 | 692588.65 | 838541.52 |

Spandrel Shear Design—Diagonal Reinforcement

| Station Location | Adiag mm² | Shear Combo | Vu N | VuLimit N | L/H Ratio | Seismic Design | Diag Reinf Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 2780 | UDWal19 | 889685.83 | 759245.52 | 1.5 | Yes | Yes |
| Right | 2781 | UDWal18 | 889776.07 | 759245.52 | 1.5 | Yes | Yes |

مشخصات تیر همبند (S2) – طبقه ششم

As=1489 mm2→USE 5∅20 Top & Bot             طولی

As=3255 mm2→USE 7∅25 Left & Right             قطری

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story7 | S2 | 24000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1325 | 0.28 | UDWal19 | -378944970 |
| Right | 1325 | 0.28 | UDWal18 | -378981827 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1262 | 0.26 | UDWal34 | 361165385 |
| Right | 1262 | 0.26 | UDWal35 | 361130064 |

Spandrel Shear Design

| Station Location | Avert mm²/mm | Ahoriz mm²/mm | ShearCombo | Vu N | ϕVc N | ϕVs N | ϕVn N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 2.07 | 1 | UDWal34 | 696548.86 | 159679.68 | 536869.18 | 696548.86 |
| Right | 2.07 | 1 | UDWal35 | 696494.24 | 159689.16 | 536805.08 | 696494.24 |

Spandrel Shear Design—Diagonal Reinforcement

| Station Location | Adiag mm² | Shear Combo | Vu N | VuLimit N | L/H Ratio | Seismic Design | Diag Reinf Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 2325 | UDWal19 | 743961.35 | 759245.52 | 1.5 | Yes | No |
| Right | 2325 | UDWal18 | 744031.98 | 759245.52 | 1.5 | Yes | No |

مشخصات تیر همبند (S2) – طبقه هفتم

ETABS 2016 Shear Wall Design

ACI 318-14  Spandrel Design

Spandrel Details

| Story ID | Spandrel ID | Centroid X (mm) | Centroid Y (mm) | Depth (mm) | Width (mm) | LLRF |
| --- | --- | --- | --- | --- | --- | --- |
| Story8 | S2 | 24000 | 23600 | 1200 | 200 | 1 |

Material Properties

| Ec (MPa) | f'c (MPa) | Lt.Wt Factor (Unitless) | fy (MPa) | fys (MPa) |
| --- | --- | --- | --- | --- |
| 27600 | 28 | 1 | 400 | 400 |

Design Code Parameters

| ΦT | ΦC | ΦV | ΦV (Seismic) |
| --- | --- | --- | --- |
| 0.9 | 0.65 | 0.75 | 0.6 |

Spandrel Flexural Design—Top Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 1025 | 0.21 | UDWal19 | -294532936 |
| Right | 1026 | 0.21 | UDWal18 | -294587300 |

Spandrel Flexural Design—Bottom Reinforcement

| Station Location | Reinf Area mm² | Reinf Percentage | Reinf Combo | Moment, Mu N-mm |
| --- | --- | --- | --- | --- |
| Left | 971 | 0.2 | UDWal34 | 279148238 |
| Right | 971 | 0.2 | UDWal35 | 279092422 |

Spandrel Shear Design

| Station Location | Avert mm²/mm | Ahoriz mm²/mm | ShearCombo | Vu N | ϕVc N | ϕVs N | ϕVn N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 1.4 | 1 | UDWal34 | 538664.27 | 176149.09 | 362515.18 | 538664.27 |
| Right | 1.4 | 1 | UDWal35 | 538596.13 | 176170.44 | 362425.69 | 538596.13 |

Spandrel Shear Design—Diagonal Reinforcement

| Station Location | Adiag mm² | Shear Combo | Vu N | VuLimit N | L/H Ratio | Seismic Design | Diag Reinf Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 1815 | UDWal19 | 580913.49 | 759245.52 | 1.5 | Yes | No |
| Right | 1816 | UDWal18 | 581054.92 | 759245.52 | 1.5 | Yes | No |

مشخصات تیر همبند (S2) – طبقه هفتم

As=1262 mm2→USE 5∅20 Top & Bot             طولی

As=2325 mm2→USE 6∅25 Left & Right             قطری