User Report 1

Model File: ETABS-6Story-Karimi-Steel, Revision 0

2/6/2018

| Table of Contents |  |
| --- | --- |
| 1. Structure Data | 3 |
| 1.1 Story Data | 3 |
| 1.2 Grid Data | 3 |
| 1.3 Point Coordinates | 3 |
| 2. Properties | 5 |
| 2.1 Materials | 5 |
| 2.2 Frame Sections | 5 |
| 3. Loads | 6 |
| 3.1 Load Patterns | 6 |
| 3.2 Load Sets | 6 |
| 3.3 Auto Seismic Loading | 6 |
| 3.4 Applied Loads | 11 |
| 3.4.1 Area Loads | 11 |
| 3.5 Load Cases | 11 |
| 3.6 Load Combinations | 11 |

# 1 Structure Data

This chapter provides model geometry information, including items such as story levels, point coordinates, and element connectivity.

## 1.1 Story Data

Table 1.1 - Story Data

| Name | Height m | Elevation m | Master Story | Similar To | Splice Story |
| --- | --- | --- | --- | --- | --- |
| Story2 | 2.5 | 5.7 | No | None | No |
| Story1 | 3.2 | 3.2 | No | None | No |
| Base | 0 | 0 | No | None | No |

## 1.2 Grid Data

Table 1.2 - Grid Systems

| Name | Type | Story Range | X Origin m | Y Origin m | Rotation deg | Bubble Size m | Color |
| --- | --- | --- | --- | --- | --- | --- | --- |
| G1 | Cartesian | Default | 0 | 0 | 0 | 1.25 | ffa0a0a0 |

Table 1.3 - Grid Lines

| Grid System | Grid Direction | Grid ID | Visible | Bubble Location | Ordinate m |
| --- | --- | --- | --- | --- | --- |
| G1 | X | A | Yes | End | 0 |
| G1 | X | B | Yes | End | 8.35 |
| G1 | X | C | Yes | End | 15.05 |
| G1 | Y | 5 | Yes | Start | 0 |
| G1 | Y | 4 | Yes | Start | 5.5 |
| G1 | Y | 3 | Yes | Start | 10.67 |
| G1 | Y | 2 | Yes | Start | 11.05 |
| G1 | Y | 1 | Yes | Start | 11.55 |

## 1.3 Point Coordinates

Table 1.4 - Joint Coordinates Data

| Label | X m | Y m | ΔZ Below m |
| --- | --- | --- | --- |
| 1 | 0 | 11.55 | 0 |
| 2 | 8.35 | 11.05 | 0 |
| 3 | 15.05 | 10.67 | 0 |
| 4 | 0 | 5.5 | 0 |
| 5 | 8.35 | 5.5 | 0 |
| 6 | 0 | 0 | 0 |
| 7 | 8.35 | 0 | 0 |
| 8 | 15.05 | 5.5 | 0 |
| 9 | 15.05 | 0 | 0 |
| 38 | 9.35 | 11.05 | 0 |
| 39 | 9.35 | 5.5 | 0 |
| 41 | 9.35 | 0 | 0 |
| 42 | 8.35 | -1 | 0 |
| 43 | 0 | -1 | 0 |
| 64 | 9.35 | -1 | 0 |

# 2 Properties

This chapter provides property information for materials, frame sections, shell sections, and links.

## 2.1 Materials

Table 2.1 - Material Properties - Summary

| Name | Type | E kgf/m² | ν | Unit Weight kgf/m³ | Design Strengths |
| --- | --- | --- | --- | --- | --- |
| A416Gr270 | Tendon | 20037484345 | 0 | 7849.05 | Fy=172322365.37 kgf/m², Fu=189828799.06 kgf/m² |
| A615Gr60 | Rebar | 20389019158 | 0.3 | 7849.05 | Fy=42184177.57 kgf/m², Fu=63276266.35 kgf/m² |
| A992Fy50 | Steel | 20389019158 | 0.3 | 7849.05 | Fy=35153481.31 kgf/m², Fu=45699525.7 kgf/m² |
| C0 | Concrete | 2534563564 | 0.2 | 0 | Fc=2812278.5 kgf/m² |
| ST37(Ry=1.15) | Steel | 20000000000 | 0.3 | 7850 | Fy=24000000 kgf/m², Fu=37000000 kgf/m² |
| ST37(Ry=1.2) | Steel | 20000000000 | 0.3 | 7850 | Fy=24000000 kgf/m², Fu=37000000 kgf/m² |

## 2.2 Frame Sections

Table 2.2 - Frame Sections - Summary

| Name | Material | Shape |
| --- | --- | --- |
| 2UPN80(D12) | ST37(Ry=1.2) | Steel Double Channel |
| Box18X18X1.2 | ST37(Ry=1.15) | Steel Tube |
| Box24X24X1.2 | ST37(Ry=1.15) | Steel Tube |
| IPE180 | ST37(Ry=1.2) | Steel I/Wide Flange |
| IPE200 | ST37(Ry=1.2) | Steel I/Wide Flange |
| IPE220 | ST37(Ry=1.2) | Steel I/Wide Flange |
| PL27X0.6-12X1.2 | ST37(Ry=1.15) | Steel I/Wide Flange |

# 3 Loads

This chapter provides loading information as applied to the model.

## 3.1 Load Patterns

Table 3.1 - Load Patterns

| Name | Type | Self Weight Multiplier | Auto Load |
| --- | --- | --- | --- |
| Dead | Dead | 1 |  |
| Live | Live | 0 |  |
| L ROOF | Roof Live | 0 |  |
| Snow | Snow | 0 |  |
| EX | Seismic | 0 | User Coefficient |
| EY | Seismic | 0 | User Coefficient |
| Mass | Other | 0 |  |
| NDEADX | Notional | 0 | Auto |
| NDEADY | Notional | 0 | Auto |
| NLIVEX | Notional | 0 | Auto |
| NLIVEY | Notional | 0 | Auto |
| NLRoofX | Notional | 0 | Auto |
| NLRoofY | Notional | 0 | Auto |
| NSX | Notional | 0 | Auto |
| NSY | Notional | 0 | Auto |

## 3.2 Load Sets

Table 3.2 - Shell Uniform Load Sets

| Load Set | Load Pattern | Load kgf/m² |
| --- | --- | --- |
| Story Load | Dead | 443 |
| Story Load | Live | 200 |
| Roof Load | Dead | 430 |
| Roof Load | L ROOF | 150 |
| Roof Load | Snow | 105 |

## 3.3 Auto Seismic Loading

# User Coefficient Auto Seismic Load Calculation

This calculation presents the automatically generated lateral seismic loads for load pattern EX using the user input coefficients, as calculated by ETABS.

### Direction and Eccentricity

Direction = X

### Factors and Coefficients

### Equivalent Lateral Forces

| Base Shear Coefficient, C | C=0.1925 |
| --- | --- |
| Base Shear, V | V=CW |

### Calculated Base Shear

| Direction | Period Used (sec) | C | W (kgf) | V (kgf) |
| --- | --- | --- | --- | --- |
| X | 0 | 0 | 137217.23 | 26414.32 |

### Applied Story Forces

| Story; Elevation; X-Dir; Y-Dir. m; kgf; kgf. Story2; 5.7; 13298.93; 0. Story1; 3.2; 13115.39; 0. Base; 0; 0; 0. |  |
| --- | --- |

# User Coefficient Auto Seismic Load Calculation

This calculation presents the automatically generated lateral seismic loads for load pattern EY using the user input coefficients, as calculated by ETABS.

### Direction and Eccentricity

Direction = Y

### Factors and Coefficients

### Equivalent Lateral Forces

| Base Shear Coefficient, C | C=0.175 |
| --- | --- |
| Base Shear, V | V=CW |

### Calculated Base Shear

| Direction | Period Used (sec) | C | W (kgf) | V (kgf) |
| --- | --- | --- | --- | --- |
| Y | 0 | 0 | 137217.23 | 24013.02 |

### Applied Story Forces

Story; Elevation; X-Dir; Y-Dir. m; kgf; kgf. Story2; 5.7; 0; 12089.93. Story1; 3.2; 0; 11923.08. Base; 0; 0; 0.

## 3.4 Applied Loads

### 3.4.1 Area Loads

Table 3.5 - Shell Loads - Uniform

| Story | Label | Unique Name | Load Pattern | Direction | Load kgf/m² |
| --- | --- | --- | --- | --- | --- |
| Story2 | F9 | 6 | Dead | Gravity | 223 |
| Story2 | F10 | 7 | Dead | Gravity | 223 |
| Story2 | F11 | 8 | Dead | Gravity | 223 |
| Story2 | F12 | 10 | Dead | Gravity | 223 |
| Story1 | F7 | 3 | Dead | Gravity | 430 |
| Story1 | F8 | 4 | Dead | Gravity | 430 |
| Story1 | F7 | 3 | Live | Gravity | 200 |
| Story1 | F8 | 4 | Live | Gravity | 200 |

Table 3.6 - Shell Loads - Uniform Load Sets

| Story | Label | Unique Name | Load Set | Load Pattern | Load kgf/m² |
| --- | --- | --- | --- | --- | --- |
| Story2 | F5 | 1 | Roof Load | Dead | 430 |
| Story2 | F5 | 1 | Roof Load | L ROOF | 150 |
| Story2 | F5 | 1 | Roof Load | Snow | 105 |
| Story2 | F6 | 2 | Roof Load | Dead | 430 |
| Story2 | F6 | 2 | Roof Load | L ROOF | 150 |
| Story2 | F6 | 2 | Roof Load | Snow | 105 |
| Story1 | F5 | 5 | Story Load | Dead | 443 |
| Story1 | F5 | 5 | Story Load | Live | 200 |

## 3.5 Load Cases

Table 3.7 - Load Cases - Summary

| Name | Type |
| --- | --- |
| Dead | Linear Static |
| Live | Linear Static |
| L ROOF | Linear Static |
| Snow | Linear Static |
| EX | Linear Static |
| EY | Linear Static |
| Mass | Linear Static |
| NDEADX | Linear Static |
| NDEADY | Linear Static |
| NLIVEX | Linear Static |
| NLIVEY | Linear Static |
| NLRoofX | Linear Static |
| NLRoofY | Linear Static |
| NSX | Linear Static |
| NSY | Linear Static |

## 3.6 Load Combinations

Table 3.8 - Load Combinations

| Name | Load Case/Combo | Scale Factor | Type | Auto |
| --- | --- | --- | --- | --- |
| UDStlS1 | Dead | 1.4 | Linear Add | No |
| UDStlS1 | NDEADX | 1.4 |  | No |
| UDStlS2 | Dead | 1.4 | Linear Add | No |
| UDStlS2 | NDEADX | -1.4 |  | No |
| UDStlS3 | Dead | 1.4 | Linear Add | No |
| UDStlS3 | NDEADY | 1.4 |  | No |
| UDStlS4 | Dead | 1.4 | Linear Add | No |
| UDStlS4 | NDEADY | -1.4 |  | No |
| UDStlS5 | Dead | 1.2 | Linear Add | No |
| UDStlS5 | Live | 1.6 |  | No |
| UDStlS5 | Snow | 0.5 |  | No |
| UDStlS5 | NDEADX | 1.2 |  | No |
| UDStlS5 | NLIVEX | 1.6 |  | No |
| UDStlS5 | NSX | 0.5 |  | No |
| UDStlS6 | Dead | 1.2 | Linear Add | No |
| UDStlS6 | Live | 1.6 |  | No |
| UDStlS6 | Snow | 0.5 |  | No |
| UDStlS6 | NDEADX | -1.2 |  | No |
| UDStlS6 | NLIVEX | -1.6 |  | No |
| UDStlS6 | NSX | -0.5 |  | No |
| UDStlS7 | Dead | 1.2 | Linear Add | No |
| UDStlS7 | Live | 1.6 |  | No |
| UDStlS7 | Snow | 0.5 |  | No |
| UDStlS7 | NDEADY | 1.2 |  | No |
| UDStlS7 | NLIVEY | 1.6 |  | No |
| UDStlS7 | NSY | 0.5 |  | No |
| UDStlS8 | Dead | 1.2 | Linear Add | No |
| UDStlS8 | Live | 1.6 |  | No |
| UDStlS8 | Snow | 0.5 |  | No |
| UDStlS8 | NDEADY | -1.2 |  | No |
| UDStlS8 | NLIVEY | -1.6 |  | No |
| UDStlS8 | NSY | -0.5 |  | No |
| UDStlS9 | Dead | 1.2 | Linear Add | No |
| UDStlS9 | Live | 1.6 |  | No |
| UDStlS9 | L ROOF | 0.5 |  | No |
| UDStlS9 | NDEADX | 1.2 |  | No |
| UDStlS9 | NLIVEX | 1.6 |  | No |
| UDStlS9 | NLRoofX | 0.5 |  | No |
| UDStlS10 | Dead | 1.2 | Linear Add | No |
| UDStlS10 | Live | 1.6 |  | No |
| UDStlS10 | L ROOF | 0.5 |  | No |
| UDStlS10 | NDEADX | -1.2 |  | No |
| UDStlS10 | NLIVEX | -1.6 |  | No |
| UDStlS10 | NLRoofX | -0.5 |  | No |
| UDStlS11 | Dead | 1.2 | Linear Add | No |
| UDStlS11 | Live | 1.6 |  | No |
| UDStlS11 | L ROOF | 0.5 |  | No |
| UDStlS11 | NDEADY | 1.2 |  | No |
| UDStlS11 | NLIVEY | 1.6 |  | No |
| UDStlS11 | NLRoofY | 0.5 |  | No |
| UDStlS12 | Dead | 1.2 | Linear Add | No |
| UDStlS12 | Live | 1.6 |  | No |
| UDStlS12 | L ROOF | 0.5 |  | No |
| UDStlS12 | NDEADY | -1.2 |  | No |
| UDStlS12 | NLIVEY | -1.6 |  | No |
| UDStlS12 | NLRoofY | -0.5 |  | No |
| UDStlS13 | Dead | 1.2 | Linear Add | No |
| UDStlS13 | Live | 1 |  | No |
| UDStlS13 | Snow | 1.6 |  | No |
| UDStlS13 | NDEADX | 1.2 |  | No |
| UDStlS13 | NLIVEX | 1 |  | No |
| UDStlS13 | NSX | 1.6 |  | No |
| UDStlS14 | Dead | 1.2 | Linear Add | No |
| UDStlS14 | Live | 1 |  | No |
| UDStlS14 | Snow | 1.6 |  | No |
| UDStlS14 | NDEADX | -1.2 |  | No |
| UDStlS14 | NLIVEX | -1 |  | No |
| UDStlS14 | NSX | -1.6 |  | No |
| UDStlS15 | Dead | 1.2 | Linear Add | No |
| UDStlS15 | Live | 1 |  | No |
| UDStlS15 | Snow | 1.6 |  | No |
| UDStlS15 | NDEADY | 1.2 |  | No |
| UDStlS15 | NLIVEY | 1 |  | No |
| UDStlS15 | NSY | 1.6 |  | No |
| UDStlS16 | Dead | 1.2 | Linear Add | No |
| UDStlS16 | Live | 1 |  | No |
| UDStlS16 | Snow | 1.6 |  | No |
| UDStlS16 | NDEADY | -1.2 |  | No |
| UDStlS16 | NLIVEY | -1 |  | No |
| UDStlS16 | NSY | -1.6 |  | No |
| UDStlS17 | Dead | 1.2 | Linear Add | No |
| UDStlS17 | Live | 1 |  | No |
| UDStlS17 | L ROOF | 1.6 |  | No |
| UDStlS17 | NDEADX | 1.2 |  | No |
| UDStlS17 | NLIVEX | 1 |  | No |
| UDStlS17 | NLRoofX | 1.6 |  | No |
| UDStlS18 | Dead | 1.2 | Linear Add | No |
| UDStlS18 | Live | 1 |  | No |
| UDStlS18 | L ROOF | 1.6 |  | No |
| UDStlS18 | NDEADX | -1.2 |  | No |
| UDStlS18 | NLIVEX | -1 |  | No |
| UDStlS18 | NLRoofX | -1.6 |  | No |
| UDStlS19 | Dead | 1.2 | Linear Add | No |
| UDStlS19 | Live | 1 |  | No |
| UDStlS19 | L ROOF | 1.6 |  | No |
| UDStlS19 | NDEADY | 1.2 |  | No |
| UDStlS19 | NLIVEY | 1 |  | No |
| UDStlS19 | NLRoofY | 1.6 |  | No |
| UDStlS20 | Dead | 1.2 | Linear Add | No |
| UDStlS20 | Live | 1 |  | No |
| UDStlS20 | L ROOF | 1.6 |  | No |
| UDStlS20 | NDEADY | -1.2 |  | No |
| UDStlS20 | NLIVEY | -1 |  | No |
| UDStlS20 | NLRoofY | -1.6 |  | No |
| UDStlS21 | Dead | 1.2 | Linear Add | No |
| UDStlS21 | Live | 1 |  | No |
| UDStlS21 | Snow | 0.2 |  | No |
| UDStlS21 | EX | 1 |  | No |
| UDStlS22 | Dead | 1.2 | Linear Add | No |
| UDStlS22 | Live | 1 |  | No |
| UDStlS22 | Snow | 0.2 |  | No |
| UDStlS22 | EX | -1 |  | No |
| UDStlS23 | Dead | 1.2 | Linear Add | No |
| UDStlS23 | Live | 1 |  | No |
| UDStlS23 | Snow | 0.2 |  | No |
| UDStlS23 | EY | 1 |  | No |
| UDStlS24 | Dead | 1.2 | Linear Add | No |
| UDStlS24 | Live | 1 |  | No |
| UDStlS24 | Snow | 0.2 |  | No |
| UDStlS24 | EY | -1 |  | No |
| UDStlS25 | Dead | 0.9 | Linear Add | No |
| UDStlS25 | EX | 1 |  | No |
| UDStlS26 | Dead | 0.9 | Linear Add | No |
| UDStlS26 | EX | -1 |  | No |
| UDStlS27 | Dead | 0.9 | Linear Add | No |
| UDStlS27 | EY | 1 |  | No |
| UDStlS28 | Dead | 0.9 | Linear Add | No |
| UDStlS28 | EY | -1 |  | No |
| UDStlD1 | Dead | 1 | Linear Add | No |
| UDStlD2 | Dead | 1 | Linear Add | No |
| UDStlD2 | Live | 1 |  | No |

| TABLE: Material List by Element Type |  |  |  |
| --- | --- | --- | --- |
| Element Type | Material | Total Weight | # Pieces |
|  |  | kgf |  |
| Column | ST37(Ry=1.15) | 3288.11 | 15 |
| Beam | ST37(Ry=1.15) | 2413.43 | 9 |
| Beam | ST37(Ry=1.2) | 1763.54 | 20 |
| Brace | ST37(Ry=1.2) | 895.65 | 8 |
| Total |  | 8360 |  |