ETABS 2016 Steel Frame Design

AISC 360-10 Steel Section Check (Strength Summary)

Element Details

| Level | Element | Unique Name | Location (cm) | Combo | Element Type | Section | Classification |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Story1 | B21 | 46 | 10 | UDStlS4 | Intermediate Moment Frame | IPE240 | Seismic HD |

LLRF and Demand/Capacity Ratio

| L (cm) | LLRF | Stress Ratio Limit |
| --- | --- | --- |
| 500.000 | 1 | 1 |

Analysis and Design Parameters

| Provision | Analysis | 2nd Order | Reduction |
| --- | --- | --- | --- |
| LRFD | Direct Analysis | General 2nd Order | Tau-b Variable |

Stiffness Reduction Factors

| αPr /Py | αPr /Pe | τb | EA factor | EI factor |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 0.8 | 0.8 |

Seismic Parameters

| Ignore Seismic Code? | Ignore Special EQ Load? | Plug Welded? | SDC | I | Rho | SDS | R | Ω0 | Cd |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | No | No | D | 1 | 1 | 1.05 | 5 | 3 | 4 |

Design Code Parameters

| Φb | Φc | ΦTY | ΦTF | ΦV | ΦV-RI | ΦVT |
| --- | --- | --- | --- | --- | --- | --- |
| 0.9 | 0.9 | 0.9 | 0.75 | 0.9 | 1 | 1 |

Section Properties

| A (cm²) | J (cm⁴) | I33 (cm⁴) | I22 (cm⁴) | Av3 (cm²) | Av2 (cm²) |
| --- | --- | --- | --- | --- | --- |
| 39.1 | 13 | 3892 | 284 | 23.5 | 14.9 |

Design Properties

| S33 (cm³) | S22 (cm³) | Z33 (cm³) | Z22 (cm³) | r33 (cm) | r22 (cm) | Cw (cm⁶) |
| --- | --- | --- | --- | --- | --- | --- |
| 324.3 | 47.3 | 367 | 73.9 | 9.977 | 2.695 | 37391.2 |

Material Properties

| E (tonf/cm²) | fy (tonf/cm²) | Ry | α |
| --- | --- | --- | --- |
| 2000 | 2.4 | 1.2 | NA |

Stress Check forces and Moments

| Location (cm) | Pu (tonf) | Mu33 (tonf-cm) | Mu22 (tonf-cm) | Vu2 (tonf) | Vu3 (tonf) | Tu (tonf-cm) |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | 0 | -1709.256 | 0 | -11.4946 | 0 | -0.035 |

Axial Force & Biaxial Moment Design Factors   (H1-1b)

|  | L Factor | K1 | K2 | B1 | B2 | Cm |
| --- | --- | --- | --- | --- | --- | --- |
| Major Bending | 0.96 | 1 | 1 | 1 | 1 | 1 |
| Minor Bending | 0 | 1 | 1 | 1 | 1 | 1 |

Parameters for Lateral Torsion Buckling

| Lltb | Kltb | Cb |
| --- | --- | --- |
| 1 | 1 | 2.362 |

Demand/Capacity (D/C) Ratio  Eqn.(H1-1b)

| D/C Ratio = | (Pr /2Pc ) + (Mr33 /Mc33 ) + (Mr22 /Mc22 ) |
| --- | --- |
| 2.156 = | 0 + 2.156 + 0 |

Axial Force and Capacities

| Pu Force (tonf) | ϕPnc Capacity (tonf) | ϕPnt Capacity (tonf) |
| --- | --- | --- |
| 0 | 61.4468 | 84.456 |

Moments and Capacities

|  | Mu Moment (tonf-cm) | ϕMn (tonf-cm) | ϕMn No LTB (tonf-cm) | ϕMn Cb=1 (tonf-cm) |
| --- | --- | --- | --- | --- |
| Major Bending | 1709.256 | 792.72 | 792.72 | 484.595 |
| Minor Bending | 0 | 159.624 |  |  |

Shear Design

|  | Vu Force (tonf) | ϕVn Capacity (tonf) | Stress Ratio |
| --- | --- | --- | --- |
| Major Shear | 11.4946 | 21.4272 | 0.536 |
| Minor Shear | 0 | 30.4819 | 0 |

End Reaction Major Shear Forces

| Left End Reaction (tonf) | Load Combo | Right End Reaction (tonf) | Load Combo |
| --- | --- | --- | --- |
| 11.4946 | UDStlS10 | 11.7491 | UDStlS10 |