ETABS 2016 Steel Frame Design

AISC 360-10 Steel Section Check (Strength Summary)

Element Details

| Level | Element | Unique Name | Location (cm) | Combo | Element Type | Section | Classification |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Story1 | C1 | 1 | 0 | UDStlS4 | Intermediate Moment Frame | TUBO200X200X15 | Seismic HD |

LLRF and Demand/Capacity Ratio

| L (cm) | LLRF | Stress Ratio Limit |
| --- | --- | --- |
| 320.000 | 1 | 1 |

Analysis and Design Parameters

| Provision | Analysis | 2nd Order | Reduction |
| --- | --- | --- | --- |
| LRFD | Direct Analysis | General 2nd Order | Tau-b Variable |

Stiffness Reduction Factors

| αPr /Py | αPr /Pe | τb | EA factor | EI factor |
| --- | --- | --- | --- | --- |
| 0.046 | 0.009 | 1 | 0.8 | 0.8 |

Seismic Parameters

| Ignore Seismic Code? | Ignore Special EQ Load? | Plug Welded? | SDC | I | Rho | SDS | R | Ω0 | Cd |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | No | No | D | 1 | 1 | 1.05 | 5 | 3 | 4 |

Design Code Parameters

| Φb | Φc | ΦTY | ΦTF | ΦV | ΦV-RI | ΦVT |
| --- | --- | --- | --- | --- | --- | --- |
| 0.9 | 0.9 | 0.9 | 0.75 | 0.9 | 1 | 1 |

Section Properties

| A (cm²) | J (cm⁴) | I33 (cm⁴) | I22 (cm⁴) | Av3 (cm²) | Av2 (cm²) |
| --- | --- | --- | --- | --- | --- |
| 111 | 9497.4 | 6373.3 | 6373.3 | 46.5 | 46.5 |

Design Properties

| S33 (cm³) | S22 (cm³) | Z33 (cm³) | Z22 (cm³) | r33 (cm) | r22 (cm) | Cw (cm⁶) |
| --- | --- | --- | --- | --- | --- | --- |
| 637.3 | 637.3 | 771.8 | 771.8 | 7.577 | 7.577 |  |

Material Properties

| E (tonf/cm²) | fy (tonf/cm²) | Ry | α |
| --- | --- | --- | --- |
| 2000 | 2.4 | 1.2 | NA |

HSS Section Parameters

| HSS Welding | Reduce HSS Thickness? |
| --- | --- |
| SAW | Yes |

Stress Check forces and Moments

| Location (cm) | Pu (tonf) | Mu33 (tonf-cm) | Mu22 (tonf-cm) | Vu2 (tonf) | Vu3 (tonf) | Tu (tonf-cm) |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | -12.2551 | -2717.399 | 202.166 | 0 | 0 | -11.815 |

Axial Force & Biaxial Moment Design Factors   (H1-1b)

|  | L Factor | K1 | K2 | B1 | B2 | Cm |
| --- | --- | --- | --- | --- | --- | --- |
| Major Bending | 0.925 | 1 | 1 | 1 | 1 | 0.381 |
| Minor Bending | 0.925 | 1 | 1 | 1 | 1 | 0.446 |

Parameters for Lateral Torsion Buckling

| Lltb | Kltb | Cb |
| --- | --- | --- |
| 0.925 | 1 | 2.18 |

Demand/Capacity (D/C) Ratio  Eqn.(H1-1b)

| D/C Ratio = | (Pr /2Pc ) + (Mr33 /Mc33 ) + (Mr22 /Mc22 ) |
| --- | --- |
| 1.779 = | 0.028 + 1.63 + 0.121 |

Axial Force and Capacities

| Pu Force (tonf) | ϕPnc Capacity (tonf) | ϕPnt Capacity (tonf) |
| --- | --- | --- |
| 12.2551 | 221.8458 | 239.76 |

Moments and Capacities

|  | Mu Moment (tonf-cm) | ϕMn (tonf-cm) | ϕMn No LTB (tonf-cm) | ϕMn Cb=1 (tonf-cm) |
| --- | --- | --- | --- | --- |
| Major Bending | 2717.399 | 1666.98 | 1666.98 | 1666.98 |
| Minor Bending | 202.166 | 1666.98 |  |  |

Torsion Moment and Capacities

| Tu Moment (tonf-cm) | Tn Capacity (tonf-cm) | ϕTn Capacity (tonf-cm) |
| --- | --- | --- |
| -11.815 | 1459.747 | 1313.772 |

Shear Design

|  | Vu Force (tonf) | ϕVn Capacity (tonf) | Stress Ratio |
| --- | --- | --- | --- |
| Major Shear | 0 | 60.264 | 0.234 |
| Minor Shear | 0 | 60.264 | 0.02 |